Package:      areas/speech/systems/celp/

Name:         CELP

Summary:      FS-1016 Code Excited Linear Prediction coder

Version:      3.2a (3-AUG-93)

Description:

CELP is a lossy compression algorithm used for low bit rate (e.g.,

4800 bps in U.S. Fed-Std-1016) speech coding. CELP stands for "code

excited linear prediction". CELP coding, at 4800 bps, breaks the

performance barrier of most Government standards, providing Consortium

ratings of "very good" intelligibility and "excellent" quality,

comparable to 32,000 bps CVSD.

Like all vector quantization techniques, CELP coding is a

frame-oriented technique that breaks a sampled input signal into

blocks of samples (i.e., vectors) that are processed as one unit.

CELP coding is based on analysis-by-synthesis search procedures,

perceptually weighted vector quantization (VQ), and linear prediction

(LP).  A 10th order LP filter is used to model the speech signal's

short-term spectrum, or formant structure.  Long-term signal

periodicity, or pitch, is modeled by an adaptive code book VQ.  The

residual from the short-term LP and pitch VQ is vector quantized using

a fixed stochastic code book.  The optimal scaled excitation vectors

from the adaptive and stochastic code books are selected by minimizing

a time varying, perceptually weighted distortion measure that improves

subjective speech quality by exploiting masking properties of human

hearing.

The CELP coder's computational requirements are dominated by the two

code book searches.  The computational complexity and speech quality

of the coder depend upon the search sizes of the code books.  Any

subset of either code book can be searched to fit processor

constraints, at the expense of speech quality.

Fed-Std-1016 uses an 8 kHz sample rate and a 30 ms frame size with

four 7.5 ms subframes.  CELP analysis consists of three basic

functions: 1) short-term linear prediction, 2) long-term adaptive code

book search, and 3) innovation stochastic code book search.  CELP

synthesis consists of the corresponding three synthesis functions

performed in reverse order with the optional addition of a fourth

function, called a postfilter, to enhance the output speech.  The

transmitted CELP parameters are the stochastic code book index and

gain, the adaptive code book index and gain, and 10 line spectral

parameters (LSP).

This directory contains the U.S. DoD's Federal-Standard-1016 based

4800 bps code excited linear prediction voice coder version 3.2

(CELP 3.2) Fortran and C simulation source codes. Example input and

processed speech files are included.

[Not really AI, but included for completeness relative to

comp.speech. --mark]

Requires:     C or Fortran

Ports:        Tested on a Sun4 (SunOS 4.1.3)

Origin:       super.org:/pub/celp_3.2a.tar.Z [192.31.192.1]

Copying:      Worldwide distribution permitted.

Updated:

CD-ROM:       Prime Time Freeware for AI, Issue 1-1

Bug Reports:

Mailing List:

Author(s):    Joe Campbell <jpcampb@afterlife.ncsc.mil>

or <74040.305@compuserve.com>

Very limited support is available from the authors.  Please do not

send questions or suggestions without first reading the documentation

(README files, the Technical Information Bulletin, etc.).

Contact:      Joe Campbell <jpcampb@afterlife.ncsc.mil>

Department of Defense

Craig F. Reese  <cfreese@super.org>

IDA/Supercomputing Research Center

Keywords:

Speech Coding, Authors!Campbell, CELP,

Speech Compression, Linear Prediction, Vector Quantization, C!Code,

Fortran!Code, Code Excited Linear Prediction

Contains:    ???

See Also:

areas/speech/systems/lpc/

areas/speech/systems/ldcelp/

References:

Technical Information Bulletin (TIB), "Details to Assist in

Implementation of Federal Standard 1016 CELP", is available through

the National Technical Information Service: NTIS, U.S. Department of

Commerce, 5285 Port Royal Road, Springfield, VA 22161, USA, (phone:

703-487-4650/4600).

The following articles describe the Federal-Standard-1016 4.8-kbps CELP

coder (it's unnecessary to read more than one):

Campbell, Joseph P. Jr., Thomas E. Tremain and Vanoy C. Welch,

"The Federal Standard 1016 4800 bps CELP Voice Coder," Digital Signal

Processing, Academic Press, 1991, Vol. 1, No. 3, p. 145-155.

Campbell, Joseph P. Jr., Thomas E. Tremain and Vanoy C. Welch,

"The DoD 4.8 kbps Standard (Proposed Federal Standard 1016),"

in Advances in Speech Coding, ed. Atal, Cuperman and Gersho,

Kluwer Academic Publishers, 1991, Chapter 12, p. 121-133.

Campbell, Joseph P. Jr., Thomas E. Tremain and Vanoy C. Welch, "The

Proposed Federal Standard 1016 4800 bps Voice Coder:  CELP," Speech

Technology Magazine, April/May 1990, p. 58-64.

The U.S. DoD's Federal-Standard-1015/NATO-STANAG-4198 based 2400 bps

linear prediction coder (LPC-10) was republished as a Federal

Information Processing Standards Publication 137 (FIPS Pub 137).

It is described in:

Thomas E. Tremain, "The Government Standard Linear Predictive Coding

Algorithm:  LPC-10," Speech Technology Magazine, April 1982, p. 40-49.

There is also a section about FS-1015 in the book:

Panos E. Papamichalis, Practical Approaches to Speech Coding,

Prentice-Hall, 1987.

The voicing classifier used in the enhanced LPC-10 (LPC-10e) is

described in:

Campbell, Joseph P., Jr. and T. E. Tremain, "Voiced/Unvoiced

Classification of Speech with Applications to the U.S. Government

LPC-10E Algorithm," Proceedings of the IEEE International Conference

on Acoustics, Speech, and Signal Processing, 1986, p. 473-6.

Copies of the official standard "Federal Standard 1016,

Telecommunications: Analog to Digital Conversion of Radio Voice by

4,800 bit/second Code Excited Linear Prediction (CELP)" are available

for $5.00 each from GSA Federal Supply Service Bureau, Specification

Section, Suite 8100, 470 E. L'Enfant Place, S.W., Washington, DC

20407, 1-202-755-0325.