Carnegie Mellon University	 CMU-MSIT-SE-RC2-CRS-0.9

School of Computer Science	Fall 2003

Master of Software Engineering

MSIT-SE Practicum

Railroad Configuration Rule Specification

Version 0.92

Railroad Configuration Rule Checker

November 12, 2003

Yong Hoon Choi

Revision History

| Date | Revision | Description | Author |
| --- | --- | --- | --- |
|  | 0.1 | Documentation Creation | Yong Hoon Choi |
|  | 0.9 |  |  |
| 11/12/2003 | 0.92 |  |  |

Table of Contents

1	Introduction	5

2	System File Structure	5

3	Language for Rule Specification	6

4	Configuration Rules	7

4.1	LC (Location) Macro	7

4.1.1	Rule LC.1	7

4.2	CP (Control Point) Macro	8

4.2.1	Rule CP.1	8

4.2.2	Rule CP.2	8

4.2.3	Rule CP.3	9

4.2.4	Rule CP.4	9

4.3	SW (Switch) Macro	10

4.3.1	Rule SW.1	10

4.3.2	Rule SW.2	10

4.3.3	Rule SW.3	11

4.3.4	Rule SW.4	11

4.3.5	Rule SW.5	12

4.3.6	Rule SW.6	12

4.4	SG (Signal) Macro	13

4.4.1	Rule SG.1	13

4.4.2	Rule SG.2	13

4.4.3	Rule SG.3	14

4.4.4	Rule SG.4	15

4.4.5	Rule SG.5	15

4.4.6	Rule SG.6	16

4.5	SL (Signal) Macro	17

4.5.1	Rule SL.1	17

4.5.2	Rule SL.2	17

4.5.3	Rule SL.3	18

4.5.4	Rule SL.4	18

4.6	RT (Route) Macro	19

4.6.1	Rule RT.1	19

4.6.2	Rule RT.2	20

4.6.3	Rule RT.3	20

4.6.4	Rule RT.4	21

4.6.5	Rule RT.5	22

4.6.6	Rule RT. 6	22

4.6.7	Rule RT.7	22

4.7	TK (Track) Macro	24

4.7.1	Rule TK.1	24

4.7.2	Rule TK.2	25

4.7.3	Rule TK.3	26

4.7.4	Rule TK.4	27

4.8	PTK (Pseudo Track) Macro	28

4.8.1	Rule PTK.1	28

4.8.2	Rule PTK.2	28

4.8.3	Rule PTK.3	29

4.8.4	Rule PTK.4	29

4.8.5	Rule PTK.5	30

4.8.6	Rule PTK.6	30

4.9	T2 Macro	31

4.9.1	Rule T2.1	31

4.10	HTK (Hotbox Track) Macro	32

4.10.1	Rule HTK.1	32

4.10.2	Rule HTK.2	32

4.10.3	Rule HTK.3	33

4.10.4	Rule HTK.4	33

4.11	TP (Track Path) Macro	34

4.11.1	Rule TP.1	34

4.11.2	Rule TP.2	35

4.11.3	Rule TP.3	36

4.11.4	Rule TP.4	37

4.11.5	Rule TP.5	38

4.11.6	Rule TP.6	39

4.11.7	Rule TP.7	40

5	Summary	41

# Introduction

The purpose of the Configuration Rule Specification Document for the Railroad Configuration Rule Checker project is to provide an overall structure of the configuration information in the tabling file as well as desirable rules for the syntactically and semantically correct configuration.

# System File Structure

The system file, which contains the railroad configuration information, is a text file consisting of VAX macros. Each VAX macro represents a logical/physical component in the railroad configuration. There are a variety of configuration components, such as LC (location) and CP (control point), which have hierarchical relationships among them. The table 1 shows the macro command scope related to configuration components and the hierarchical relationships.

Location (LC)

Control Point (CP)

Switch (SW)

Signal (SG)

Signal Lamp (SL)

Route (RT)

Track (TK)

Pseudo Track (PTK)

Hotbox Track (HTK)

Track Length  (T2)

Parallel Track (PARTK)

Track Path (TP)

Table 1. Hierarchical relationships of configuration components

# Language for Rule Specification

The language for configuration rules is defined as follows:

BNF(Backus Naur Form) SPECS ::= { SPEC } SPEC ::= “(“ <VARIABLE> <PREDICATE> ARGS “)” ARGS ::= { <ARG> } LEXICAL ELEMENTS <VARIABLE> = <COMPONENT NAME>[.<ELEMENT NAME>] <PREDICATE> = [a-zA-Z]+[a-zA-Z0-9_]* <COMPONENT NAME> = [a-zA-Z]+[a-zA-Z0-9_]* <ELEMENT NAME> = [a-zA-Z]+[a-zA-Z0-9_]* <ARG> = [a-zA-Z]+[a-zA-Z0-9_]*

# Configuration Rules

This part of the document will provide 1) informal description, 2) syntax, and 3) semantics of the configuration rules applied to each configuration component.  The informal description of the rules will be represented in English while the semantics of the rules will be represented in a first-order predicate logic. In the syntax description, the reserved words will be bold-fonted.

The process for configuration rules will be performed as shown below.

Configuration

Rule

Checking

Engine

“OK”

Message

True

False

Error Message

+

Macro Location

## LC (Location) Macro

The LC component has the following fields in order:

| Index | Field Name | Description |
| --- | --- | --- |
| 1 | INLC | Internal Location Number |
| 2 | LFTLC | Next Location to the Left |
| 3 | RGTLC | Next Location to the Right |
| 4 | LCNAM | Location Name |
| 5 | SCN | Detail Screen Name |

### Rule LC.1

- Description

The first argument (INLC) is always incremented by one from the previous location macro.

- Syntax

(LC.INLC INCREMENT_BY_ONE)

- Semantics

(FORALL a IN LC, (FORALL b IN LC, (IF ((a.INDEX + 1) = b.INDEX) THEN ((a.INLC + 1) = b.INLC) ) ) )

## CP (Control Point) Macro

The CP component has the following fields in order:

| Index | Field Name | Description |
| --- | --- | --- |
| 1 | CPN | Relative CP Number at Location |
| 2 | EXLN | External Location Number |
| 3 | TERN | Territory Number |
| 4 | LCTYP | Location Type |
| 5 | CPNAM | Control Point Name |
| 6 | TIMZ | Time Zone Bias |
| 7 | SEC | Section CP is found in |
| 8 | MP | Mile Post at this CP |
| 9 | CPSEQ | Sequence of CP relative to location |

### Rule CP.1

- Description

The CP macro re-starts at one after every location macro.

- Syntax

(CP.CPN START_FROM_ONE)

- Semantics

(FORALL a IN LC, (EXISTS b IN a.CP, ( (b.INDEX = 0) AND (b.CPN = 1) ) ) )

### Rule CP.2

- Description

The first argument is always incremented by one from the previous CP macro.

- Syntax

(CP.CPN INCREMENT_BY_ONE)

- Semantics

(FORALL a IN LC (FORALL b IN a.CP (FORALL c IN a.CP (IF ((b.INDEX + 1) = c.INDEX) THEN ((b.INLC + 1) = c.INLC) ) ) ) )

### Rule CP.3

- Description

The sixth argument (TIMZ) is always a 0, 1, or 2.

- Syntax

(CP.TIMZ AMONG 0 1 2)

- Semantics

(FORALL a IN LC (FORALL b IN a.CP ( (b.TIMZ = 0) OR (b.TIMZ = 1) OR (b.TIMZ = 2) ) ) )

### Rule CP.4

- Description

If there is a ninth argument it can NOT be greater than the number of CP’s inside that location.

- Syntax

(CP.CPSEQ <= CP.NUMBER)

- Semantics

(FORALL a IN LC (FORALL b in a.CP (b.CPSEQ <= a.CP.size) ) )

## SW (Switch) Macro

The SW component has the following fields in order:

| Index | Field Name | Description |
| --- | --- | --- |
| 1 | INS | Internal Switch Number |
| 2 | EXS | External Switch Number |
| 3 | CPSW | CP of Switch |
| 4 | SWTYP | Switch Type |
| 5 | LTK | Left Track Number |
| 6 | RTK | Right Track Number |
| 7 | SWSPD | Switch Speed |

### Rule SW.1

- Description

The SW macro re-starts at 1 after every location macro.

- Syntax

(SW.INS START_FROM_ONE)

- Semantics

(FORALL a IN LC (EXISTS b IN a.SW (IF (b.INDEX = 0) THEN (b.INS = 1) ) ) )

### Rule SW.2

- Description

The first argument is always incremented by one from the previous SW macro.

- Syntax

(SW.INS INCREMENT_BY_ONE)

- Semantics

(FORALL a IN LC (FORALL b IN a.SW (FORALL c IN a.SW (IF ((b.INDEX + 1) = c.INDEX) THEN ((b.INS + 1) = c.INS) ) ) ) )

### Rule SW.3

- Description

The second argument (with the exception of zero or no number present) is always odd.

- Syntax

(SW.EXS ODD)

- Semantics

(FORALL a IN LC (FORALL b IN a.SW ((b.EXS % 2) = 1) ) )

### Rule SW.4

- Description

The third argument (CPSW: CP of switch) points to a CP.  There must be a CP macro of that number inside this location and the first number of the second argument (EXLN) of that CP must not be zero.  In addition, the fourth argument of that CP must not be PSDCP.

- Syntax

(SW CP_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN a.SW (EXISTS c IN a.CP ( (b.CPSW = c.CPN) AND (c.EXLN.CHAR(0) != ‘0’) AND (c.LCTYP != PSDCP) ) ) ) )

### Rule SW.5

- Description

The fifth argument (LTK) points to the track on the left.  If the number here is not zero at least one of the fifth arguments (i.e. one of the arguments in the SW list, SWL) in that corresponding TK macro must be the first argument of that switch (INS).  If the switch is on a track, the track must have a left track connected to it.

- Syntax

(SW.LTK TR_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN a.SW (IF (b.LTK != NULL) THEN (EXISTS c IN b.TK, ( (b.LTK = c.TKN) AND (b.INS IN c.SWL) ) ) ) ) )

### Rule SW.6

- Description

The sixth argument (RTK) points to the right track.  If the number here is not zero at least one of the fifth arguments in SWL of the corresponding TK macro must be the first argument of that switch. If the switch is on a track, the track must have a right  track connected to it.

- Syntax

(SW.RTK TR_CORRESPONDENCE)

- Semantics

(FORALL a IN LC, (FORALL b IN a.SW, (IF (b.RTK != NULL) THEN (EXISTS c IN b.TK, ( (b.RTK = c.TKN) AND (b.INS IN c.SWL) ) ) ) )

## SG (Signal) Macro

The SG component has the following fields in order:

| Index | Field Name | Description |
| --- | --- | --- |
| 1 | SGN | Internal Signal Number |
| 2 | SGTYP | Signal Type |
| 3 | STK | Signal’s Track |
| 4 | CPSG | CP for Signal |
| 5 | LHB | Location Hot Box |
| 6 | CPHB | Control Point of Hot Box |

### Rule SG.1

- Description

The SG macro re-starts at 1 after every location macro.

- Syntax

(SG.SGN, START_FROM_ONE)

- Semantics

(FORALL a IN LC (EXISTS b IN a.SG (IF (b.INDEX = 0) THEN (b.SGN = 1) ) ) )

### Rule SG.2

- Description

The first argument is always incremented by one from the previous SG macro.

- Syntax

(SG.SGN INCREMENT_BY_ONE)

- Semantics

(FORALL a IN LC (FORALL b IN a.SG (FORALL c IN a.SG (IF ((b.INDEX + 1) = c.INDEX) THEN ((b.SGN + 1) = c.SGN) ) ) ) )

### Rule SG.3

- Description

If there is a third argument (STK: signal’s track), it must appear in at least one of the TP’s underneath the corresponding track (LTK or RTK).  If the first argument of the SG (SGN) is odd if will be the eighth argument (LSL: path’s left signal lamp number) of that TP.  If it is even, it will be the ninth argument (RSL: path’s right signal lamp number) of that TP.

(If the signal is on a track, it should be on one of the track paths. If it’s a NE signal it will be signal to the left, otherwise it will be signal to the right)

There will be exceptions to this rule.

- Syntax

(SG.STK TP_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN a.SG (EXIST c IN a.TK ( (b.STK = c.TKN) AND (EXIST d IN c.TP ( (IF ((b.SGN % 2) = 1) THEN (b.SGN = d.LSL) ) AND (IF ((b.SGN % 2) = 0) THEN (b.SGN = d.RSL) ) ) ) ) ) ) )

### Rule SG.4

- Description

The fourth argument (CPSG: CP for Signal) points to the owning CP inside this location.  There must be a corresponding CP inside this location of that argument. The first number of the second argument (EXLN) of that CP can not be zero.

- Syntax

(SG.CPSG CP_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN a.SG (EXISTS c IN a.CP) ( (c.CPN = b.CPSG) AND (c.EXLN.CHAR(0) != ‘0’) ) ) )

### Rule SG.5

- Description

If there is a fifth argument (LHB: Location Hotbox) there should also be a sixth argument (CPHB: Control Point of Hotbox).

- Syntax

(SG LMB COME_WITH CPHB)

- Semantics

(FORALL a IN LC (FORALL b IN a.SG, (IF (SG.LHB != NULL) THEN (SG.CPHB != NULL) ) ) )

### Rule SG.6

- Description

The fifth (LHB: location hotbox)/sixth (CPHB: control point of hotbox) arguments combine to form a hotbox location/CP combination.  There must be a valid CP at the location pointed to by the fifth argument.  The fourth argument (LCTYP: location type) of that CP should be HBXCP.  The first number of the second argument (EXLN: external location number) of that CP can not be zero.

- Syntax

(SG HBX_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN a.SG, (EXISTS c IN LC ( (b.LHB = c.INLC) AND (EXISTS d IN c.CP ( (b.CPHB = d.CPN) AND (d.LCTYP = HBXCP) AND (d.EXLN.CHAR(0) != ‘0’) ) ) ) ) ) )

## SL (Signal) Macro

The SL component has the following fields in order:

| Index | Field Name | Description |
| --- | --- | --- |
| 1 | SLN | Internal Signal Lamp Number |
| 2 | EXS | External Signal Lamp Number |
| 3 | CPSL | CP Controlling Signal Lamp |

### Rule SL.1

- Description

The SL macro re-starts at 1 after every LC macro.

- Syntax

(SL.SLN START_FROM_ONE)

- Semantics

(FORALL a IN LC (EXISTS b IN a.SL (IF (b.INDEX = 0) THEN (b.SLN = 1) ) ) )

### Rule SL.2

- Description

The first argument is always incremented by one from the previous SL macro.

- Syntax

(SG.SGN INCREMENT_BY_ONE)

- Semantics

(FORALL a IN LC (FORALL b IN a.SL (FORALL c IN a.SL (IF ((b.INDEX + 1) = c.INDEX) THEN ((b.SLN + 1) = c.SLN) ) ) ) )

### Rule SL.3

- Description

If there is a second argument (EXS: external signal lamp number), it must be even.

- Syntax

(SL.EXS EVEN)

- Semantics

(FORALL a IN LC (FORALL b IN a.SL ((b.EXS % 2) = 0) ) )

### Rule SL.4

- Description

The third argument (CPSL: CP controlling signal lamp) points to a CP.  There must be a valid CP of that number inside this location.  The first number of the second argument (EXLN: external location number) of that CP can not be zero.

- Syntax

(SL CP_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN a.SL (EXISTS c IN a.CP ( (b.CPSL = c.CPN) AND (c.EXLN.CHAR(0) != ‘0’) ) ) )

## RT (Route) Macro

The RT component has the following fields in order:

| Index | Field Name | Description |  |
| --- | --- | --- | --- |
| 1 | RTNO | Relative Route Number |  |
| 2 | RTETYP | Route Type |  |
| 3 | EXITLC | Location of Exit Signal Lamp |  |
| 4 | EXITSL | Relative Exit Signal Lamp for Route |  |
| 5 | RTLIST | DEV | Relative Device Number (Signal Lamp Number for Signal Clear) |
|  |  | REQ | Device Request To Line Route |

### Rule RT.1

- Description

The RT macro re-starts at 1 after every SL macro.

- Syntax

(RT.RTNO START_FROM_ONE)

- Semantics

(FORALL a IN LC (FORALL b IN a.SL (EXISTS c IN b.RT (IF (c.INDEX = 0) THEN (c.RTNO = 1) ) ) ) )

### Rule RT.2

- Description

The first argument is always incremented by one from the previous RT macro.

- Syntax

(RT.RTNO INCREMENT_BY_ONE)

- Semantics

(FORALL a IN LC (FORALL b IN a.SL (FORALL c IN b.RT (FORALL d IN b.RT (IF ((c.INDEX + 1) = d.INDEX) THEN ((c.RTNO + 1) = d.RTNO) ) ) ) ) )

### Rule RT.3

- Description

If there is a third argument (EXITLC: location of exit signal lamp), there must be a fourth argument (EXITSL: relative exit signal lamp for route).

- Syntax

(RT.EXITLC COME_WITH EXITSL)

- Semantics

(FORALL a IN LC (FORALL b IN a.SL (FORALL c IN b.RT (IF (c.EXITLC != NULL) THEN (c.EXITSL != NULL) ) ) ) )

### Rule RT.4

- Description

The third (EXITLC: location of exit signal lamp)/fourth (relative exit signal lamp for route) argument combine to form a location/SL There must be a valid SL at the third argument’s location. If there is no third argument the location is considered to be the current location.

- Syntax

(RT.EXITLC SL_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN a.SL (FORALL c IN b.RT (EXISTS d IN LC ( ( (c.EXITLC = d.INLC) OR (b.INLC = d.INLC) ) AND (EXISTS e IN d.SL (c.EXITSL = e.SLN) ) ) ) ) ) )

### Rule RT.5

- Description

The fourth argument (EXITSL: relative exit signal lamp for route) is the exit signal lamp.  There must be a corresponding route back from the exit signal lamp. The SW arguments called out in the 5th argument or route list should be the same arguments called out in the 5th argument of the return route although they might be in different order.

- Syntax

(RT.EXITSL SL_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN a.SL (FORALL c IN b.RT (EXISTS d IN a.SL ( (c.EXITSL = d.SLN) AND (EXISTS e IN d.RT (b.SLN = e.EXITSL) ) ) ) ) ) )

### Rule RT. 6

- Description

The last argument of the route list should be a <XX,SC>  where XX is the SL number we’re currently working under.

- Syntax

(RT.RTLIST SL_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN SL (FORALL c IN b.RT (RT.RTLIST.LAST.DEV = b.SLN) ) ) )

### Rule RT.7

- Description

There must be an equal number of ‘<’ and ‘>’ in the route list.

- Syntax

(RT.RTLIST PARANTHESIS_CHECK)

- Semantics (N/A)

## TK (Track) Macro

The TK component has the following fields in order:

| Index | Field Name | Description |
| --- | --- | --- |
| 1 | TKN | Internal Track Number |
| 2 | EXTKNO | External Number for Track |
| 3 | CPTK | CP for Track |
| 4 | TKTYUP | Track Type |
| 5 | SWL | Switch List |
| 6 | CORLC | Corresponding Location |
| 7 | CORTK | Corresponding Track |

### Rule TK.1

- Description

The TK macro re-starts at 1 after every LC macro.

- Syntax

(TK.TKN START_FROM_ONE)

- Semantics

(FORALL a IN LC (EXISTS b IN a.TK (IF (b.INDEX = 0) THEN (b.TKN = 1) ) ) )

### Rule TK.2

- Description

The first argument is always incremented by one from the previous TK or PTK macro.

- Syntax

(TK.TKN INCREMENT_BY_ONE)

- Semantics

(FORALL a IN LC (FORALL b IN a.TK (FORALL c IN a.TK (FORALL d IN a.PTK (FORALL e IN a.PTK ( (IF ((b.INDEX + 1) = c.INDEX) THEN ((b.TKN + 1) = c.TKN) ) OR (IF ((b.INDEX + 1) = d.INDEX) THEN ((b.TKN + 1) = d.TKN) ) OR (IF ((d.INDEX + 1) = c.INDEX) THEN ((d.TKN + 1) = c.TKN) ) OR (IF ((d.INDEX + 1) = e.INDEX) THEN ((d.TKN + 1) = e.TKN) ) ) ) ) ) ) )

### Rule TK.3

- Description

The third argument (CPTK: CP for track) points to the CP.  There must be a valid CP of that number inside this location.  The first number of the second argument (EXLN: external location number) of that CP can not be zero.

- Syntax

(TK CP_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN a.TK (EXISTS c IN a.CP ( (b.CPTK = c.CPN) AND (c.EXLN.CHAR(0) != ‘0’) ) ) ) )

### Rule TK.4

- Description

If the fourth argument (TKTYUP) has the DUMTK flag set in it.  (dumltf, dumrtf, trndum or dumtk) and there is a 6th (CORLC) and 7th argument (CORTK), there must be a corresponding track (7th argument) at the location (6th argument).  That track can not have a DUMTK flag in its track type  (4th argument).

- Syntax

(TK DUMTK_TK_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN a.TK (IF ( ( (b.TKTYUP = DUMLTF) OR (b.TKTYUP = DUMRTF) OR (b.TKTYUP = TRNDUM) OR (b.TKTYUP = DUMTK) ) AND ( (b.CORLC != NULL) AND (b.CORTK != NULL) ) ) THEN (EXISTS c IN LC (EXISTS d IN c.TK ( (b.CORLC = c.INLC) AND (b.CORTK = d.TKN) AND ( (d.TKTYUP = DUMLTF) OR (d.TKTYUP = DUMRTF) OR (d.TKTYUP = TRNDUM) OR (d.TKTYUP = DUMTK) ) ) ) ) ) ) )

## PTK (Pseudo Track) Macro

The PTK component has the following fields in order:

| Index | Field Name | Description |
| --- | --- | --- |
| 1 | TKN | Internal Track Number |
| 2 | EXTKNO | External Track Number for Track |
| 3 | CPTK | CP for Track |
| 4 | TKTYP | Track Type |
| 5 | PSD | Pseudo Track Number |

### Rule PTK.1

- Description

The PTK is unique in that it is a macro that is kept in sequence the same as the TK macro.  The first argument is always incremented by one from the previous TK or PTK macro.

- Syntax

(PTK.TKN INCREMENT_BY_ONE)

- Semantics

This rule is semantically same as the rule TK.2.

### Rule PTK.2

- Description

There must be a second argument in the PTK macro.

- Syntax

(PTK.EXTKNO REQUIRED)

- Semantics

(FORALL a IN LC (FORALL b IN a.PTK (b.EXTKNO != NULL) ) )

### Rule PTK.3

- Description

The second argument in the PTK macro should be greater than 60.

- Syntax

(PTK.EXTKNO > 60)

- Semantics

(FORALL a IN LC (FORALL b IN a.PTK (b.EXTKNO > 60) ) )

### Rule PTK.4

- Description

There must be a third argument (CPTK: CP for track) in the PTK macro.  This points to the relative CP.  There must be a valid CP of that number inside this location.  The first number of the second argument (EXLN: external location number) of that CP can not be zero.

- Syntax

(PTK CP_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN a.PTK (EXISTS c IN a.CP ( (b.CPTK = c.CPN) AND (c.EXLN.CHAR(0) != ‘0’) ) ) ) )

### Rule PTK.5

- Description

The fourth argument must be PSDENT or PSDEAP.

- Syntax

(PTK.TKTYP AMONG PSDENT PSDEAP)

- Semantics

(FORALL a IN LC (FORALL b IN a.PTK ( (b.TKTYP = PSDENT) OR (b.TKTYP = PSDEAP) ) ) )

### Rule PTK.6

- Description

The fifth argument (PSD: pseudo track number) points to a relative track inside the current location.  There must be a TK with that number that does not have a DUMTK flag set in the fourth argument (TKTYUP: track type).  There must also be a TP macro associated with that TK.

- Syntax

(PTK TK_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN a.PTK (FORALL c IN a.TK ( (b.PSD = c.TKN) AND ( (c.TKTYUP != DUMLTF) AND (c.TKTYUP = DUMRTF) AND (c.TKTYUP = TRNDUM) AND (c.TKTYUP = DUMTK) ) AND (c.TP.SIZE != 0) ) ) ) )

## T2 Macro

The T2 component has the following fields in order:

| Index | Field Name | Description |
| --- | --- | --- |
| 1 | TKLN | Track Length |

### Rule T2.1

- Description

This is the track length macro. There is only one argument here and it must be numeric.

- Syntax

(T2.TKLN REQUIRED NUMBERIC)

- Semantics (N/A)

## HTK (Hotbox Track) Macro

The HTK component has the following fields in order:

| Index | Field Name | Description |
| --- | --- | --- |
| 1 | HXNAM | Hotbox Name |
| 2 | HXMP | Hotbox Mile Post |
| 3 | HXCP | Relative CP to Send for Hotbox Track |
| 4 | HXLC | Relative Internal location to Send |
| 5 | HXTK | Relative Track At Hotbox Location |
| 6 | HXDF | Default Hotbox All Clear |

### Rule HTK.1

- Description

The second argument (HXMP) needs to be six characters long.  It consists of three numbers, a decimal point and then two other numbers, respectively.

- Syntax

(HTK.HXMP HXMP_FORMAT)

- Semantics (N/A)

### Rule HTK.2

- Description

The third argument (HXCP: relative CP to send for hotbox track) is the relative CP.  This needs to be the first argument of a CP macro whose fourth argument (LCTYP: location type) is HBXCP.

- Syntax

(HTK CP_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN TK (FORALL c IN HTK (EXIST d IN a.CP ( (c.HXCP = d.CPN) AND (d.LCTYP = HBXCP) ) ) ) ) )

### Rule HTK.3

- Description

The fourth (HXLC: relative internal location to send) /fifth arguments (HXTK: relative track at hotbox location) are a location/track.  The fourth argument should be the same as the first argument of the current location.

- Syntax

(HTK LC_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN TK (FORALL c IN b.HTK (c.HXLC = a.INLC) ) ) )

### Rule HTK.4

- Description

The fifth argument (HXTK: relative track at hotbox location) should be the same as the first argument (TKN: internal track number)of the preceding TK.

- Syntax

(HTK TK_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN TK (FORALL c IN b.HTK (c.HXTK = b.TKN) ) ) )

## TP (Track Path) Macro

The TP component has the following fields in order:

| Index | Field Name | Description |  |
| --- | --- | --- | --- |
| 1 | RTP | Relative Track Path Number |  |
| 2 | PTYP | Path Type |  |
| 3 | SWP | Switch Positions |  |
|  |  | NR | Normal Right |
|  |  | NL | Normal Left |
|  |  | RV | Reverse |
|  |  | UN | Unused |
| 4 | LTK | Path’s Left Track Number |  |
| 5 | RTK | Path’s Right Track Number |  |
| 6 | LSG | Path’s Left Signal Number |  |
| 7 | RSG | Path’s Right Signal Number |  |
| 8 | LSL | Path’s Left Signal Lamp Number |  |
| 9 | RSL | Path’s Right Signal Lamp Number |  |
| 10 | LLC | Path’s Left Location/Zone |  |
| 11 | RLC | Path’s Right Location/Zone |  |

### Rule TP.1

- Description

The TP macro re-starts at 1 after every TK macro.

- Syntax

(TP.RTP START_FROM_ONE)

- Semantics

(FORALL a IN LC (FORALL b IN a.TK (EXISTS c IN b.TP (IF (c.INDEX = 0) THEN (c.RTP = 1) ) ) ) )

### Rule TP.2

- Description

The number of arguments in the switch list, which is the third argument (SWP: switch positions) should be same as the number of switches called out in the preceding TK macro’s fifth argument (SWL: switch list).

- Syntax

(TP TK_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN TK (FORALL c IN TP ( ( (c.SWP = UN) AND (b.SWL.SIZE = 0) ) OR ( (c.SWP != UN) AND (c.SWP.SIZE = b.SWL.SIZE) ) ) ) ) )

### Rule TP.3

- Description

The fourth argument (LTK: path’s left track number) and the tenth argument (LLC: path’s left location/zone) combine form a track/location.  The same is true for the fifth (RTK: path’s right track number) and eleventh argument (RLC: path’s right location/zone).  There should be corresponding paths back to each TK from the location/track called out in these macros.  If track 1 at location 1 has track 3 at location 2 to the right there should be a corresponding TP at location 2 track 3 that would show location 1 track 1 to the left.

- Syntax

(TP TK_LC_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN TK (FORALL c IN TP (EXISTS d IN LC (EXISTS e IN TK (EXISTS f IN TP ( ( (d.INLC = c.LLC) AND (e.TKN = c.LTK) AND (a.INLC = f.RLC) AND (b.TKN = f.RTK) ) OR ( (d.INLC = c.RLC) AND (e.TKN = c.RTK) AND (a.INLC = f.LLC) AND (b.TKN = f.LTK) ) ) ) ) ) ) ) )

### Rule TP.4

- Description

The sixth argument (LSG: path’s left signal number) is the signal to the left.  There should be a valid SG macro in this location whose first argument (SGN: internal signal number) is that number and whose third argument (STK: signal’s track) is the same as the first argument (TKN: internal track number) of the preceding TK.

- Syntax

(TP.LSG SG_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN TK (FORALL c IN TP ( (c.LSG = 0) OR (EXISTS d IN SG ( (c.LSG = d.SGN) AND (d.STK != b.TKN) ) ) ) ) ) )

### Rule TP.5

- Description

The seventh argument (RSG: path’s right signal number) is the signal to the right.  There should be a valid SG macro in this location whose first argument is that number and whose third argument is the same as the preceding TK.

- Syntax

(TP.RSG SG_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN TK (FORALL c IN TP ( (c.RSG = 0) OR (EXISTS d IN SG ( (c.RSG = d.SGN) AND (d.STK != b.TKN) ) ) ) ) ) )

### Rule TP.6

- Description

The eighth argument (LSL: path’s left signal lamp number) is the signal lamp to the left.  There should be a valid SL macro in this location that corresponds to this number.  The third argument (CPSL: CP controlling signal lamp) of that (the SL) macro must not be blank.

- Syntax

(TP.LSL SL_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN TK (FORALL c IN TP ( (c.LSG = 0) OR (EXISTS d IN SL ( (c.LSG = d.SLN) AND (d.CPSL != NULL) ) ) ) ) ) )

### Rule TP.7

- Description

The ninth argument (RSL: path’s right signal lamp number) is the signal lamp to the right.  There should be a valid SL macro in this location that corresponds to this number if it is not zero.  The third argument (CPSL: CP controlling signal lamp) of that (the SL) macro must not be blank.

- Syntax

(TP.RSL SL_CORRESPONDENCE)

- Semantics

(FORALL a IN LC (FORALL b IN TK (FORALL c IN TP ( (c.RSG = 0) OR (EXISTS d IN SL ( (c.RSG = d.SLN) AND (d.CPSL != NULL) ) ) ) ) ) )

# Summary

There are various predicates which have different semantics according to the configuration component that they applied to. The table 1 shows the relationships between predicates and configuration components.

| Component Predicate | LC | CP | SW | SG | SL | RT | TK | PTK | T2 | HTK | TP | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INCREMENT_BY_ONE | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  |  |  | 8 |
| START_FROM_ONE |  | 1 | 1 | 1 | 1 | 1 | 1 |  |  |  | 1 | 7 |
| AMONG |  | 1 |  |  |  |  |  | 1 |  |  |  | 2 |
| <= |  | 1 |  |  |  |  |  | 1 |  |  |  | 2 |
| EVEN |  |  |  |  | 1 |  |  |  |  |  |  | 1 |
| ODD |  |  | 1 |  |  |  |  |  |  |  |  | 1 |
| REQUIRED |  |  |  |  |  |  |  | 1 |  |  |  | 1 |
| REQUIRED NUMBERIC |  |  |  |  |  |  |  |  | 1 |  |  | 1 |
| COME_WITH |  |  |  | 1 |  | 1 |  |  |  |  |  | 2 |
| PARANTHESIS_CHECK |  |  |  |  |  | 1 |  |  |  |  |  | 1 |
| HXMP_FORMAT |  |  |  |  |  |  |  |  |  | 1 |  | 1 |
| CP_CORRESPONDENCE |  |  | 1 | 1 | 1 |  | 1 | 1 |  | 1 |  | 6 |
| TR_CORRESPONDENCE |  |  | 2 |  |  |  |  |  |  |  |  | 2 |
| TP_CORRESPONDENCE |  |  |  | 1 |  |  |  |  |  |  |  | 1 |
| HBX_CORRESPONDENCE |  |  |  | 1 |  |  |  |  |  |  |  | 1 |
| LC_CORRESPONDENCE |  |  |  |  |  |  |  |  |  | 1 | 1 | 2 |
| SG_CORRESPONDENCE |  |  |  |  |  |  |  |  |  |  | 2 | 2 |
| SL_CORRESPONDENCE |  |  |  |  |  | 2 |  |  |  |  | 2 | 4 |
| DUMTK_TK_CORRESPONDENCE |  |  |  |  |  |  | 1 |  |  |  |  | 1 |
| TK_CORRESPONDENCE |  |  |  |  |  |  |  | 1 |  | 1 | 1 | 3 |
| TK_LC_CORRESPONDENCE |  |  |  |  |  |  |  |  |  |  | 1 | 0 |
| TOTAL | 1 | 4 | 6 | 6 | 4 | 6 | 4 | 6 | 1 | 4 | 8 | 50 |

Table 2. Predicate-Component Relationships