## LC (Location) Macro

The LC component has the following fields in order:

| Index | Field Name | Description |
| --- | --- | --- |
| 1 | INLC | Internal Location Number |
| 2 | LFTLC | Next Location to the Left |
| 3 | RGTLC | Next Location to the Right |
| 4 | LCNAM | Location Name |
| 5 | SCN | Detail Screen Name |

### Rule LC.1

- Description

The first argument (INLC) is always incremented by one from the previous location macro.

- Syntax

(LC.INLC INCREMENT_BY_ONE)

- Tabling File Input

- The first argument is not incremented by one.

| Line# | Macros |
| --- | --- |
| 32491 | LC 121,0,122,<INDEPENDEN JCT>,D2121C |
| 32725 | LC 122,121,124,<LEE'S SUMMIT>,D2122A |
| 32979 | LC 124,122,125,<PLEASANT HILL>,D2124A |

## CP (Control Point) Macro

The CP component has the following fields in order:

| Index | Field Name | Description |
| --- | --- | --- |
| 1 | CPN | Relative CP Number at Location |
| 2 | EXLN | External Location Number |
| 3 | TERN | Territory Number |
| 4 | LCTYP | Location Type |
| 5 | CPNAM | Control Point Name |
| 6 | TIMZ | Time Zone Bias |
| 7 | SEC | Section CP is found in |
| 8 | MP | Mile Post at this CP |
| 9 | CPSEQ | Sequence of CP relative to location |

### Rule CP.1

- Description

The CP macro re-starts at one after every location macro.

- Syntax

(CP.CPN START_FROM_ONE)

- Tabling File Input

1) The CP macro does not start at one after every location macro.

| Line# | Macros |
| --- | --- |
| 2388 | LC 1,232,2,<GREEN RIVER>,D2001U |
| 2395 | CP 2,7814,1,XMTSLT,<E GREEN RIVER>,1,,,8147 |
| 2396 | CP 3,7809,1,PSDCP,<KANDA>,1,,,8083 |

### Rule CP.2

- Description

The first argument is always incremented by one from the previous CP macro.

- Syntax

(CP.CPN INCREMENT_BY_ONE)

- Tabling File Input

- The first argument is not incremented by one from the previous CP macro.

| Line# | Macros |
| --- | --- |
| 2607 | LC 2,1,175,<ROCK SPRINGS>,D2002G |
| 2609 | CP 1,7804,1,,<W ROCK SPRINGS>,1,,,8040,2 |
| 2610 | CP 2,7803,1,,<C ROCK SPRINGS>,1,,,8025,3 |
| 2612 | CP 4,7798,1,,<CHEVRON>,1,,,7987,5 |
| 2614 | CP 5,7808,1,HBXCP,<HBD G808>,1,,,8082,1 |

### Rule CP.3

- Description

The sixth argument (TIMZ) is always a 0, 1, or 2.

- Syntax

(CP.TIMZ AMONG 0 1 2)

- Tabling File Input

1) The sixth argument is neither 0, 1, nor 2.

| Line# | Macros |
| --- | --- |
| 2610 | CP 2,7803,1,,<C ROCK SPRINGS>,6,,,8025,3 |

### Rule CP.4

- Description

If there is a ninth argument it can NOT be greater than the number of CP’s inside that location.

- Syntax

(CP.CPSEQ <= CP.NUMBER)

- Tabling File Input

- If the ninth argument of the CP is greater than the number of CP’s inside that location.

| Line# | Macros |
| --- | --- |
| 3023 | CP 4,23768,2,ABS251,<WC BLACK BUTTE>,1,,,7669,7 |
| 3025 | CP 5,23765,2,HBXCP,<HBD W765>,1,,,7650,3 |

## SW (Switch) Macro

The SW component has the following fields in order:

| Index | Field Name | Description |
| --- | --- | --- |
| 1 | INS | Internal Switch Number |
| 2 | EXS | External Switch Number |
| 3 | CPSW | CP of Switch |
| 4 | SWTYP | Switch Type |
| 5 | LTK | Left Track Number |
| 6 | RTK | Right Track Number |
| 7 | SWSPD | Switch Speed |

### Rule SW.1

- Description

The SW macro re-starts at 1 after every location macro.

- Syntax

(SW.INS START_FROM_ONE)

- Tabling File Input

- The SW macro doesn’t start from one  after every location macro.

| Line# | Macros |
| --- | --- |
| 3016 | LC 3,170,4,<BLACK BUTTE>,D2003 |
| 3028 | SW 2,3,3,NOWHIND,8,9,40 |
| 3029 | SW 3,,2,SW251,5,0 |
| 3030 | SW 4,,5,SW251,0,15 |

### Rule SW.2

- Description

The first argument is always incremented by one from the previous SW macro.

- Syntax

(SW.INS INCREMENT_BY_ONE)

- Tabling File Input

- The first argument is incremented by two from the previous SW macro.

| Line# | Macros |
| --- | --- |
| 3309 | SW 1,1,1,,2,0 |
| 3310 | SW 2,,2,SPRING,0,5 |
| 3316 | SW 4,,1,SPRING,13,0 |
| 3317 | SW 5,3,2,,0,11 |

### Rule SW.3

- Description

The second argument (with the exception of zero or no number present) is always odd.

- Syntax

(SW.EXS ODD)

- Tabling File Input

- The second argument is even.

| Line# | Macros |
| --- | --- |
| 3767 | SW 1,2,1,,2,0 |

### Rule SW.4

- Description

The third argument (CPSW: CP of switch) points to a CP.  There must be a CP macro of that number inside this location and the first number of the second argument (EXLN) of that CP must not be zero.  In addition, the fourth argument of that CP must not be PSDCP.

- Syntax

(SW CP_CORRESPONDENCE)

- Tabling File Input

- The SW has no corresponding CP of that number inside the location.

| Line# | Macros |
| --- | --- |
| 3745 | LC 5,4,171,<TABLE ROCK>,D2005A |
| 3760 | CP 1,23746,2,XMTSLT,<W TABLE ROCK>,1,,,7465,3 |
| 3762 | CP 3,23741,2,NWHXMT,<HOLD SIGNAL>,1,,,7412,5 |
| 3768 | SW 2,,2,SPRING,0,5 |

- The first number of the second argument of the CP is zero.

| Line# | Macros |
| --- | --- |
| 3292 | LC 4,3,5,<BITTER CREEK>,D2004C |
| 3301 | CP 1,023758,2,XMTSLT,<BITTER CREEK>,1,,,7585,2 |
| 3309 | SW 1,1,1,,2,0 |

- The fourth argument of that CP is PSDCP.

| Line# | Macros |
| --- | --- |
| 3937 | LC 6,174,7,<RINER>,D2006A |
| 3942 | CP 1,23702,3,PSDCP,<W RINER>,1,,,7021 |
| 3947 | SW 1,1,1,,4,0,40 |

### Rule SW.5

- Description

The fifth argument (LTK) points to the track on the left.  If the number here is not zero at least one of the fifth arguments (i.e. one of the arguments in the SW list, SWL) in that corresponding TK macro must be the first argument of that switch (INS).  If the switch is on a track, the track must have a left track connected to it.

- Syntax

(SW.LTK TR_CORRESPONDENCE)

- Tabling File Input

- The SW’s left track doesn’t exist.

| Line# | Macros |
| --- | --- |
| 7195 | SW 1,1,1,,12,0 |
| 7258 | TK 8,3,2,DUMRTF,,22,4 |
| 7262 | TK 10,81,2,DUMTK,,22,20 |

- The SW’s left track doesn’t have the internal switch number of the SW in its SW list.

| Line# | Macros |
| --- | --- |
| 3768 | SW 1,1,1,,2,0 |
| 3848 | TK 2,62,1,B251IOS,<3> |

### Rule SW.6

- Description

The sixth argument (RTK) points to the right track.  If the number here is not zero at least one of the fifth arguments in SWL of the corresponding TK macro must be the first argument of that switch. If the switch is on a track, the track must have a right  track connected to it.

- Syntax

(SW.RTK TR_CORRESPONDENCE)

- Tabling File Input

- The SW’s right track doesn’t exist.

| Line# | Macros |
| --- | --- |
| 7196 | SW 2,3,2,,0,13 |
| 7258 | TK 8,3,2,DUMRTF,,22,4 |
| 7262 | TK 10,81,2,DUMTK,,22,20 |

- The SW’s left track doesn’t have the internal switch number of the SW in its SW list.

| Line# | Macros |
| --- | --- |
| 3769 | SW 2,,2,SPRING,0,5 |
| 3868 | TK 5,25,2,R251IOS,<2> |

### SG (Signal) Macro

The SG component has the following fields in order:

| Index | Field Name | Description |
| --- | --- | --- |
| 1 | SGN | Internal Signal Number |
| 2 | SGTYP | Signal Type |
| 3 | STK | Signal’s Track |
| 4 | CPSG | CP for Signal |
| 5 | LHB | Location Hot Box |
| 6 | CPHB | Control Point of Hot Box |

### Rule SG.1

- Description

The SG macro re-starts at 1 after every location macro.

- Syntax

(SG.SGN, START_FROM_ONE)

- Tabling File Input

- The SG macro doesn’t start from one after every location macro.

| Line# | Macros |
| --- | --- |
| 6525 | LC 21,20,22,<LARAMIE>,D2021G |
| 6562 | SG 2,SWHHCO,3,1,21,7 |
| 6564 | SG 3,NECO,4,1 |

### Rule SG.2

- Description

The first argument is always incremented by one from the previous SG macro.

- Syntax

(SG.SGN INCREMENT_BY_ONE)

- Tabling File Input

- The first argument is not incremented by one from the previous SG macro.

| Line# | Macros |
| --- | --- |
| 6333 | LC 20,19,21,<BOSLER HOLD>,D2020A |
| 6361 | SG 1,NEHHO,3,1,19,3 |
| 6362 | SG 2,SWHHO,3,1,21,6 |
| 6364 | SG 4,SWHHO,4,1,21,6 |

### Rule SG.3

- Description

If there is a third argument (STK: signal’s track), it must appear in at least one of the TP’s underneath the corresponding track (LTK or RTK).  If the first argument of the SG (SGN) is odd it will be the eighth argument (LSL: path’s left signal lamp number) of that TP.  If it is even, it will be the ninth argument (RSL: path’s right signal lamp number) of that TP.

(If the signal is on a track, it should be on one of the track paths. If it’s a NE signal it will be signal to the left, otherwise it will be signal to the right)

- Syntax

(SG.STK TP_CORRESPONDENCE)

- Tabling File Input

- The SG has no corresponding track.

| Line# | Macros |
| --- | --- |
| 6361 | SG 1,NEHHO,3,1,19,3 |
| 6410 | TK 2,21,1,BLKTK |
| 6417 | TK 4,,1,OS |

- The first argument of the SG is odd and there is no corresponding TP whose eighth argument is same as the first argument of the SG.

| Line# | Macros |
| --- | --- |
| 6361 | SG 1,NEHHO,3,1,19,3 |
| 6414 | TK 3,,1,OS |
| 6415 | TP 1,,UN,1,5,1,2,10,2,20,20 |

- The first argument of the SG is even and there is no corresponding TP whose ninth argument is same as the first argument of the SG.

| Line# | Macros |
| --- | --- |
| 6364 | SG 4,SWHHO,4,1,21,6 |
| 6417 | TK 4,,1,OS |
| 6418 | TP 1,,UN,2,6,3,4,3,14,20,20 |

### Rule SG.4

- Description

The fourth argument (CPSG: CP for Signal) points to the owning CP inside this location.  There must be a corresponding CP inside this location of that argument. The first number of the second argument (EXLN) of that CP can not be zero.

- Syntax

(SG.CPSG CP_CORRESPONDENCE)

- Tabling File Input

- There is no corresponding CP of the SG inside the location.

| Line# | Macros |
| --- | --- |
| 6525 | LC 21,20,22,<LARAMIE>,D2021G |
| 6538 | CP 5,23562,9,PSDCP,<FORELLE>,1,,,5610,6 |
| 6539 | CP 6,23576,8,HBXCP,<HBD W576>,1,,,5761,1 |
| 6540 | CP 7,23561,10,HBXCP,<HBD W561>,1,,,5616,5 |
| 6578 | SG 14,SWCO,10,10 |

- The first number of the second argument of the corresponding CP of the SG is zero.

| Line# | Macros |
| --- | --- |
| 6525 | LC 21,20,22,<LARAMIE>,D2021G |
| 6536 | CP 3,023565,9,XMTSLT,<LARAMIE>,1,,,5654,4 |
| 6589 | SG 15,NECO,17,3 |

### Rule SG.5

- Description

If there is a fifth argument (LHB: Location Hotbox) there should also be a sixth argument (CPHB: Control Point of Hotbox).

- Syntax

(SG LMB COME_WITH CPHB)

- Tabling File Input

1)  The SG has the fifth argument but no sixth argument.

| Line# | Macros |
| --- | --- |
| 6565 | SG 4,SWHHCO,4,1,21 |

### Rule SG.6

- Description

The fifth (LHB: location hotbox)/sixth (CPHB: control point of hotbox) arguments combine to form a hotbox location/CP combination.  There must be a valid CP at the location pointed to by the fifth argument.  The fourth argument (LCTYP: location type) of that CP should be HBXCP.  The first number of the second argument (EXLN: external location number) of that CP can not be zero.

- Syntax

1)

(SG HBX_CORRESPONDENCE)

?

- Tabling File Input

| Line# | Macros |
| --- | --- |
| 3942 | CP 1,23702,3,XMTSLT,<W RINER>,1,,,7021 |
| 3943 | CP 2,23700,3,XMTSLT,<E RINER>,1,,,6996 |
| 3945 | CP 3,23692,3,HBXCP,<HBD W692>,1,,,6922 |
| 3954 | SG 1,NEHHCO,4,1,174,4 |
| 3958 | SG 5,NEHHCO,7,1,174,4 |

## SL (Signal) Macro

The SL component has the following fields in order:

| Index | Field Name | Description |
| --- | --- | --- |
| 1 | SLN | Internal Signal Lamp Number |
| 2 | EXS | External Signal Lamp Number |
| 3 | CPSL | CP Controlling Signal Lamp |

### Rule SL.1

- Description

The SL macro re-starts at 1 after every LC macro.

- Syntax

(SL.SLN START_FROM_ONE)

- Tabling File Input

1) The first argument of the SL doesn’t start from one after every LC macro.

| Line# | Macros |
| --- | --- |
| 7185 | LC 22,21,23,<RED BUTTE>,D2022G |
| 7209 | SL 2,4,1 |
| 7212 | SL 3,6,1 |

### Rule SL.2

- Description

The first argument is always incremented by one from the previous SL macro.

- Syntax

(SG.SGN INCREMENT_BY_ONE)

- Tabling File Input

1) The first argument is not incremented by on from the previous SL macro.

| Line# | Macros |
| --- | --- |
| 7327 | LC 23,22,28,<HERMOSA>,D2023A |
| 7388 | SL 1 |
| 7390 | SL 2,4,2 |
| 7399 | SL 4,8,2 |

### Rule SL.3

- Description

If there is a second argument (EXS: external signal lamp number), it must be even.

- Syntax

(SL.EXS EVEN)

- Tabling File Input

- The second argument of the SL is odd.

| Line# | Macros |
| --- | --- |
| 7409 | SL 6,13,2 |

### Rule SL.4

- Description

The third argument (CPSL: CP controlling signal lamp) points to a CP.  There must be a valid CP of that number inside this location.  The first number of the second argument (EXLN: external location number) of that CP can not be zero.

- Syntax

(SL CP_CORRESPONDENCE)

- Tabling File Input

- The SL has no valid corresponding CP inside the location.

| Line# | Macros |
| --- | --- |
| 13295 | CP 1,23354,18,PSDCP,<BIG SPRINGS>,1,,,3536,1 |
| 13297 | CP 3,23344,18,PSDCP,<CP W344>,1,,,3439,5 |
| 13336 | SL 4,8,2 |

## RT (Route) Macro

The RT component has the following fields in order:

| Index | Field Name | Description |  |
| --- | --- | --- | --- |
| 1 | RTNO | Relative Route Number |  |
| 2 | RTETYP | Route Type |  |
| 3 | EXITLC | Location of Exit Signal Lamp |  |
| 4 | EXITSL | Relative Exit Signal Lamp for Route |  |
| 5 | RTLIST | DEV | Relative Device Number (Signal Lamp Number for Signal Clear) |
|  |  | REQ | Device Request To Line Route |

### Rule RT.1

- Description

The RT macro re-starts at 1 after every SL macro.

- Syntax

(RT.RTNO START_FROM_ONE)

- Tabling File Input

- The first argument of the RT does not start from one after every SL macro.

| Line# | Macros |
| --- | --- |
| 7434 | SL 12,24,3 |
| 7436 | RT 2,,,10,<<3,SWN>,<4,SWR>,<12,SGC>> |

### Rule RT.2

- Description

The first argument is always incremented by one from the previous RT macro.

- Syntax

(RT.RTNO INCREMENT_BY_ONE)

- Tabling File Input

- The first argument is not incremented by one from the previous RT macro.

| Line# | Macros |
| --- | --- |
| 7655 | SL 5,10,2 |
| 7656 | RT 1,,,4,<<2,SWN>,<5,SGC>> |
| 7657 | RT 3,ONETN,,6,<<2,SWR>,<5,SGC>> |

### Rule RT.3

- Description

If there is a third argument (EXITLC: location of exit signal lamp), there must be a fourth argument (EXITSL: relative exit signal lamp for route).

- Syntax

(RT.EXITLC COME_WITH EXITSL)

- Tabling File Input

- The RT has a third argument but no fourth argument.

| Line# | Macros |
| --- | --- |
| 14320 | RT 1,,3,,<<17,SGC>> |

### Rule RT.4

- Description

The third (EXITLC: location of exit signal lamp)/fourth (relative exit signal lamp for route) argument combine to form a location/SL. There must be a valid SL at the third argument’s location. If there is no third argument the location is considered to be the current location.

- Syntax

(RT.EXITLC SL_CORRESPONDENCE)

- Tabling File Input

fgfgf

| Line# | Macros |
| --- | --- |
| 13328 | SL 2,4,2 |
| 13330 | RT 2,,,9,<<2,SWN>,<1,SWR>,<2,SGC>> |
| 13346 | SL 7,14,4 |
| 13349 | SL 8,16,4 |

### Rule RT.5

- Description

The fourth argument (EXITSL: relative exit signal lamp for route) is the exit signal lamp.  There must be a corresponding route back from the exit signal lamp. The SW arguments called out in the 5th argument or route list should be the same arguments called out in the 5th argument of the return route although they might be in different order.

- Syntax

(RT.EXITSL SL_CORRESPONDENCE)

- Tabling File Input

- The RT has no corresponding exit signal lamp.

| Line# | Macros |
| --- | --- |
| 4495 | RT 1,DBLTK,,15,<<7,SWN>,<11,SGC>> |
| 4497 | SL 12,24,3 |
| 4501 | SL 13,26,3 |

- The RT has the corresponding exit signal lamp, but the fourth argument of the RT is not same as the fourth argument of a RT underneath the corresponding SL.

| Line# | Macros |
| --- | --- |
| 4449 | SL 1,2,1 |
| 4450 | RT 1,,,2,<<2,SWN>,<1,SGC>> |
| 4452 | SL 2,4,1 |
| 4453 | RT 1,,,1,<<2,SWN>,<1,SWN>,<2,SGC>> |

### Rule RT. 6

- Description

The last argument of the route list should be a <XX,SC>  where XX is the SL number we’re currently working under.

- Syntax

(RT.RTLIST SL_CORRESPONDENCE)

- Tabling File Input

- The first item of the last pair of the rout list of the RT is not the SL number right above.

| Line# | Macros |
| --- | --- |
| 47858 | SL 3,6,1 |
| 47859 | RT 1,,,2,<<1,SWN>,<2,SWR>,<4,SWN>,<5,SWN>,<5,SGC>> |

### Rule RT.7

- Description

There must be an equal number of ‘<’ and ‘>’ in the route list.

- Syntax

(RT.RTLIST PARANTHESIS_CHECK)

- Tabling File Input

| Line# | Macros |
| --- | --- |
| 13571 | RT 1,DBLTK,,2,<<<1,SWN>,<2,SWN>,<3,SWN>,<1,SGC>> |

## TK (Track) Macro

The TK component has the following fields in order:

| Index | Field Name | Description |
| --- | --- | --- |
| 1 | TKN | Internal Track Number |
| 2 | EXTKNO | External Number for Track |
| 3 | CPTK | CP for Track |
| 4 | TKTYUP | Track Type |
| 5 | SWL | Switch List |
| 6 | CORLC | Corresponding Location |
| 7 | CORTK | Corresponding Track |

### Rule TK.1

- Description

The TK macro re-starts at 1 after every LC macro.

- Syntax

(TK.TKN START_FROM_ONE)

- Tabling File Input

- The first argument of the TK does not start from one after every LC macro.

| Line# | Macros |
| --- | --- |
| 7986 | LC 27,26,65,<EMKAY>,D2027C |
| 8026 | TK 2,,1,OS,<1> |
| 8030 | TK 3,2,2,RTF,,27,7 |

### Rule TK.2

- Description

The first argument is always incremented by one from the previous TK or PTK macro.

- Syntax

(TK.TKN INCREMENT_BY_ONE)

- Tabling File Input

- The first argument is not incremented by one from the previous TK or PTK macro.

| Line# | Macros |
| --- | --- |
| 7908 | TK 1,1,1,RTF,,25,6 |
| 7916 | TK 3,2,2,RTF,,26,7 |
| 7923 | TK 4,3,1,SIDTK |

### Rule TK.3

- Description

The third argument (CPTK: CP for track) points to the CP.  There must be a valid CP of that number inside this location.  The first number of the second argument (EXLN: external location number) of that CP can not be zero.

- Syntax

(TK CP_CORRESPONDENCE)

- Tabling File Input

- The TK has no valid corresponding CP.

| Line# | Macros |
| --- | --- |
| 7627 | CP 1,23550,10,NOSWHTR,<W PERKINS>,1,,,5502 |
| 7628 | CP 2,23548,10,NOSWHTR,<E PERKINS>,1,,,5488 |
| 7668 | TK 2,,4,OS,<1> |

- The first number of the second argument of the CP corresponding to the TK is zero.

| Line# | Macros |
| --- | --- |
| 7748 | CP 1,023544,10,,<W HARRIMAN>,1,,,5434,2 |
| 7796 | TK 2,,1,OS,<1> |

### Rule TK.4

- Description

If the fourth argument (TKTYUP) has the DUMTK flag set in it.  (dumltf, dumrtf, trndum or dumtk) and there is a 6th (CORLC) and 7th argument (CORTK), there must be a corresponding track (7th argument) at the location (6th argument).  That track can not have a DUMTK flag in its track type  (4th argument).

- Syntax

(TK DUMTK_TK_CORRESPONDENCE)

- Tabling File Input

- The TK is a dummy track and the corresponding track and location do not exist.

| Line# | Macros |
| --- | --- |
| 7746 | LC 25,24,26,<HARRIMAN>,D2025U |
| 7826 | TK 9,1,3,DUMTK,,26,9 |
| 7869 | LC 26,25,27,<LYNCH>,D2026Z |
| 7934 | TK 6,1,2,DUMLTF,,27,1 |
| 7936 | TK 7,2,1,DUMLTF,,26,3 |

- The TK is a dummy track and the corresponding track is also a dummy track.

| Line# | Macros |
| --- | --- |
| 7986 | LC 27,26,65,<EMKAY>,D2027C |
| 8046 | TK 6,1,2,DUMTK,,65,1 |
| 16679 | LC 65,27,32,<SPEER>,D2065C |
| 16806 | TK 1,1,1,DUMTK |

## PTK (Pseudo Track) Macro

The PTK component has the following fields in order:

| Index | Field Name | Description |
| --- | --- | --- |
| 1 | TKN | Internal Track Number |
| 2 | EXTKNO | External Track Number for Track |
| 3 | CPTK | CP for Track |
| 4 | TKTYP | Track Type |
| 5 | PSD | Pseudo Track Number |

### Rule PTK.1

- Description

The PTK is unique in that it is a macro that is kept in sequence the same as the TK macro.  The first argument is always incremented by one from the previous TK or PTK macro.

- Syntax

(PTK.TKN INCREMENT_BY_ONE)

- Tabling File Input

- The first argument of the TK or the PTK is not incremented by one from the previous TK or PTK macro.

| Line# | Macros |
| --- | --- |
| 8048 | TK 7,2,1,DUMLTF,,27,3 |
| 8052 | PTK 8,93,1,PSDENT,4 |
| 8055 | TK 10,3,3,DUMTK,,27,4 |

### Rule PTK.2

- Description

There must be a second argument in the PTK macro.

- Syntax

(PTK.EXTKNO REQUIRED)

- Tabling File Input

- The PTK has no second argument.

| Line# | Macros |
| --- | --- |
| 8241 | PTK 15,,2,PSDENT,8 |

### Rule PTK.3

- Description

The second argument in the PTK macro should be greater than 60.

- Syntax

(PTK.EXTKNO > 60)

- Tabling File Input

- The second argument of the PTK is less than 60.

| Line# | Macros |
| --- | --- |
| 6972 | PTK 33,53,3,PSDENT,12 |

### Rule PTK.4

- Description

There must be a third argument (CPTK: CP for track) in the PTK macro.  This points to the relative CP.  There must be a valid CP of that number inside this location.  The first number of the second argument (EXLN: external location number) of that CP can not be zero.

- Syntax

(PTK CP_CORRESPONDENCE)

- Tabling File Input

- There is no third argument in the PTK.

| Line# | Macros |
| --- | --- |
| 7822 | PTK 8,93,,PSDENT,4 |

- The PTK has no valid corresponding CP.

| Line# | Macros |
| --- | --- |
| 3942 | CP 1,23702,3,XMTSLT,<W RINER>,1,,,7021 |
| 3943 | CP 2,23700,3,XMTSLT,<E RINER>,1,,,6996 |
| 4112 | PTK 18,83,4,PSDENT,5 |

- The first number of the second argument of the CP corresponding to the PTK is zero.

| Line# | Macros |
| --- | --- |
| 5601 | CP 1,023624,6,XMTSLT,<W MEDICINE BOW>,1,,,6245,3 |
| 5705 | PTK 13,92,1,PSDENT,8 |

### Rule PTK.5

- Description

The fourth argument must be PSDENT or PSDEAP.

- Syntax

(PTK.TKTYP AMONG PSDENT PSDEAP)

- Tabling File Input

- The fourth argument is neither PSDENT nor PSDEAP.

| Line# | Macros |
| --- | --- |
| 6046 | PTK 13,82,2,PSDENTS,7 |

### Rule PTK.6

- Description

The fifth argument (PSD: pseudo track number) points to a relative track inside the current location.  There must be a TK with that number that does not have a DUMTK flag set in the fourth argument (TKTYUP: track type).  There must also be a TP macro associated with that TK.

- Syntax

(PTK TK_CORRESPONDENCE)

- Tabling File Input

- The PTK has no valid corresponding TK.

| Line# | Macros |
| --- | --- |
| 6293 | PTK 7,92,2,PSDENT,13 |
| 6299 | TK 10,11,1,DUMTK,,20,1 |
| 6231 | TK 11,83,1,DUMTK,,20,13 |

- The PTK has the corresponding TK whose fourth argument is DUMTK.

| Line# | Macros |
| --- | --- |
| 6404 | TK 1,11,1,DUMTK |
| 6467 | PTK 13,83,1,PSDENT,1 |

- The PTK has the corresponding TK that has no associated TP.

| Line# | Macros |
| --- | --- |
| 6811 | TK 11,13,2,BLKTK |
| 6812 | T2 9419 |
| 6822 | TK 12,23,2,BLKTK |
| 6823 | T2 8949 |
| 6827 | TP 1,,UN,9,18,0,0,0,0,21,21 |
| 6968 | PTK 32,81,3,PSDENT,11 |

## T2 Macro

The T2 component has the following fields in order:

| Index | Field Name | Description |
| --- | --- | --- |
| 1 | TKLN | Track Length |

### Rule T2.1

- Description

This is the track length macro. There is only one argument here and it must be numeric.

- Syntax

(T2.TKLN REQUIRED NUMBERIC)

- Tabling File Input

- The T2 doesn’t have only one argument.

| Line# | Macros |
| --- | --- |
| 2763 | T2 55059, A |

- The T2 have non-numeric argument.

| Line# | Macros |
| --- | --- |
| 2791 | T2 SEVEN |

## HTK (Hotbox Track) Macro

The HTK component has the following fields in order:

| Index | Field Name | Description |
| --- | --- | --- |
| 1 | HXNAM | Hotbox Name |
| 2 | HXMP | Hotbox Mile Post |
| 3 | HXCP | Relative CP to Send for Hotbox Track |
| 4 | HXLC | Relative Internal location to Send |
| 5 | HXTK | Relative Track At Hotbox Location |
| 6 | HXDF | Default Hotbox All Clear |

### Rule HTK.1

- Description

The second argument (HXMP) needs to be six characters long.  It consists of three numbers, a decimal point and then two other numbers, respectively.

- Syntax

(HTK.HXMP HXMP_FORMAT)

- Tabling File Input

- The HTK’s second argument has an illegal format.

| Line# | Macros |
| --- | --- |
| 2771 | HTK KANDAE,808.203,5,2,2 |

### Rule HTK.2

- Description

The third argument (HXCP: relative CP to send for hotbox track) is the relative CP.  This needs to be the first argument of a CP macro whose fourth argument (LCTYP: location type) is HBXCP.

- Syntax

(HTK CP_CORRESPONDENCE)

- Tabling File Input

- The HTK has no valid corresponding CP.

| Line# | Macros |
| --- | --- |
| 3023 | CP 4,23768,2,ABS251,<WC BLACK BUTTE>,1,,,7669,2 |
| 3025 | CP 5,23765,2,HBXCP,<HBD W765>,1,,,7650,3 |
| 3242 | HTK BLKBUT,767.70,6,3,15 |

- The fourth argument of the CP, corresponding to the HTK, is not HBXCP.

| Line# | Macros |
| --- | --- |
| 3305 | CP 3,23755,2, NWHXMT,<HBD W755>,1,,,7547,4 |
| 3670 | HTK BITCRK,754.00,3,4,19 |

### Rule HTK.3

- Description

The fourth (HXLC: relative internal location to send) /fifth arguments (HXTK: relative track at hotbox location) are a location/track.  The fourth argument should be the same as the first argument of the current location.

- Syntax

(HTK LC_CORRESPONDENCE)

- Tabling File Input

- The fourth argument of the HTK is not same as the current location number.

| Line# | Macros |
| --- | --- |
| 11011 | LC 43,33,44,<ARCHER>,D2043B |
| 11148 | HTK ARCHR1,499.00,4,44,5 |
| 11384 | LC 44,43,45,<BURNS>,D2044B |

### Rule HTK.4

- Description

The fifth argument (HXTK: relative track at hotbox location) should be the same as the first argument (TKN: internal track number) of the preceding TK.

- Syntax

(HTK TK_CORRESPONDENCE)

- Tabling File Input

- The fifth argument of the HTK is not same as the preceding track number.

| Line# | Macros |
| --- | --- |
| 11191 | TK 11,11,2 |
| 11193 | HTK HILLS1,488.00,3,43,12 |
| 11200 | TK 12,21,2 |

## TP (Track Path) Macro

The TP component has the following fields in order:

| Index | Field Name | Description |  |
| --- | --- | --- | --- |
| 1 | RTP | Relative Track Path Number |  |
| 2 | PTYP | Path Type |  |
| 3 | SWP | Switch Positions |  |
|  |  | NR | Normal Right |
|  |  | NL | Normal Left |
|  |  | RV | Reverse |
|  |  | UN | Unused |
| 4 | LTK | Path’s Left Track Number |  |
| 5 | RTK | Path’s Right Track Number |  |
| 6 | LSG | Path’s Left Signal Number |  |
| 7 | RSG | Path’s Right Signal Number |  |
| 8 | LSL | Path’s Left Signal Lamp Number |  |
| 9 | RSL | Path’s Right Signal Lamp Number |  |
| 10 | LLC | Path’s Left Location/Zone |  |
| 11 | RLC | Path’s Right Location/Zone |  |

### Rule TP.1

- Description

The TP macro re-starts at 1 after every TK macro.

- Syntax

(TP.RTP START_FROM_ONE)

- Tabling File Input

- The TP number does not start from one after the TK macro.

| Line# | Macros |
| --- | --- |
| 11178 | TK 9,,2,OS,<3,4> |
| 11180 | TP 2,,<NR,RV>,7,10,5,0,5,0,43,43 |
| 11181 | TP 3,,<RV,NL>,10,11,0,6,0,6,43,43 |

### Rule TP.2

- Description

The number of arguments in the switch list, which is the third argument (SWP: switch positions) should be same as the number of switches called out in the preceding TK macro’s fifth argument (SWL: switch list).

- Syntax

(TP TK_CORRESPONDENCE)

- Tabling File Input

- The number of items in the third argument of the TP is not same as the number of items in the fifth argument of the preceding TK.

| Line# | Macros |
| --- | --- |
| 2507 | TK 11,,2,OS,<5,6> |
| 2508 | TP 1,,<NR,NL>,9,13,7,8,8,9,1,1 |
| 2509 | TP 2,,<NR,RV,SK>,9,12,7,0,8,0,1,1 |

### Rule TP.3

- Description

The fourth argument (LTK: path’s left track number) and the tenth argument (LLC: path’s left location/zone) combine form a track/location.  The same is true for the fifth (RTK: path’s right track number) and eleventh argument (RLC: path’s right location/zone).  There should be corresponding paths back to each TK from the location/track called out in these macros.  If track 1 at location 1 has track 3 at location 2 to the right there should be a corresponding TP at location 2 track 3 that would show location 1 track 1 to the left.

- Syntax

(TP TK_LC_CORRESPONDENCE)

- Tabling File Input

- The TP has no valid corresponding left track.

| Line# | Macros |
| --- | --- |
| 2388 | LC 1,232,2,<GREEN RIVER>,D2001U |
| 2511 | TP 4,,<RV,RV>,18,12,0,0,0,0,1,1 |
| 2522 | TK 14,21,2,LTF,,2,2 |
| 2528 | TK 16,95,1,PSDAPP |

- The TP has no valid corresponding right track.

| Line# | Macros |
| --- | --- |
| 2388 | LC 1,232,2,<GREEN RIVER>,D2001U |
| 2487 | TP 2,,<RV>,7,20,0,2,0,2,1,1 |
| 2522 | TK 14,21,2,LTF,,2,2 |
| 2528 | TK 16,95,1,PSDAPP |

### Rule TP.4

- Description

The sixth argument (LSG: path’s left signal number) is the signal to the left.  There should be a valid SG macro in this location whose first argument (SGN: internal signal number) is that number and whose third argument (STK: signal’s track) is the same as the first argument (TKN: internal track number) of the preceding TK.

- Syntax

(TP.LSG SG_CORRESPONDENCE)

- Tabling File Input

- The TP has no corresponding left SG.

| Line# | Macros |
| --- | --- |
| 8985 | SG 9,NEDMY,11,1 |
| 8985 | SG 10,SWCO,11,1 |
| 9096 | TK 7,,1,OS,<3,2,6,7,11> |
| 9097 | TP 1,,<NR,NL,NR,NL,NL>,3,16,11,6,5,6,32,32 |

- The third argument of the left SG corresponding to the TP is not same as the preceding track number of the TP.

| Line# | Macros |
| --- | --- |
| 12466 | SG 7,NECO,9,2 |
| 12529 | TK 8,,2,OS,<1,2> |
| 12530 | TP 1,,<NL,NR>,6,10,7,8,7,8,49,49 |

### Rule TP.5

- Description

The seventh argument (RSG: path’s right signal number) is the signal to the right.  There should be a valid SG macro in this location whose first argument is that number and whose third argument is the same as the preceding TK.

- Syntax

(TP.RSG SG_CORRESPONDENCE)

- Tabling File Input

- The TP has no corresponding right SG.

| Line# | Macros |
| --- | --- |
| 12665 | SG 5,NEHHCO,7,3,50,4 |
| 12666 | SG 6,SWHHCO,7,3,51,7 |
| 12720 | TP 1,,<NR,NR,NL>,5,8,3,8,2,3,50,50 |

- The third argument of the right SG corresponding to the TP is not same as the preceding track number of the TP.

| Line# | Macros |
| --- | --- |
| 12830 | SG 4,SWCO,6,1 |
| 12902 | TK 4,,1,OS,<1,2> |
| 12903 | TP 1,,<NL,NR>,2,6,3,4,3,4,51,51 |

### Rule TP.6

- Description

The eighth argument (LSL: path’s left signal lamp number) is the signal lamp to the left.  There should be a valid SL macro in this location that corresponds to this number.  The third argument (CPSL: CP controlling signal lamp) of that (the SL) macro must not be blank.

- Syntax

(TP.LSL SL_CORRESPONDENCE)

- Tabling File Input

- The TP has no corresponding left SL.

| Line# | Macros |
| --- | --- |
| 13134 | SL 12,24,2 |
| 13138 | SL 13,26,2 |
| 13160 | TP 1,,<NL,NL>,1,10,1,2,15,2,52,52 |

- The TP has the corresponding left SL whose third argument is blank.

| Line# | Macros |
| --- | --- |
| 13324 | SL 1,2, |
| 13380 | TP 1,,<NR,NL>,1,5,1,2,1,2,53,53 |

### Rule TP.7

- Description

The ninth argument (RSL: path’s right signal lamp number) is the signal lamp to the right.  There should be a valid SL macro in this location that corresponds to this number if it is not zero.  The third argument (CPSL: CP controlling signal lamp) of that (the SL) macro must not be blank.

- Syntax

(TP.RSL SL_CORRESPONDENCE)

- Tabling File Input

- The TP has no corresponding right SL.

| Line# | Macros |
| --- | --- |
| 13595 | SL 7,14,2 |
| 13599 | SL 8,16,2 |
| 13621 | TP 1,,<NR,NL,NL>,1,6,1,2,1,10,54,54 |

- The TP has the corresponding right SL whose third argument is blank.

| Line# | Macros |
| --- | --- |
| 13767 | SL 6,12, |
| 13814 | TP 1,,<NR,NL>,7,11,5,6,5,6,55,55 |