.MT2

.MB0

.PO0

.PN0

Virtual Control Program Interface

Version 1.0

June 12, 1989

VCPI SPONSORS

A.I. Architects, Inc., Peabody, MA

Lotus Development Corp., Cambridge, MA

Phar Lap Software, Inc., Cambridge, MA

Quadram, Norcross, GA

Qualitas, Inc., Bethesda, MD

Quarterdeck Office Systems, Santa Monica, CA

Rational Systems, Inc., Natick, MA

VCPI Co-ordinators:               Phar Lap Software, Inc.

(617) 661-1510

Robert Moote

Quarterdeck Office Systems

(310) 392-9851

Daniel Spear

Copyright Phar Lap Software, Inc., 1987-1989

Copyright Quarterdeck Office Systems, 1987-1989

This specification may be duplicated provided it is reproduced in its

entirety, including the above copyright notice.

Version 1.0 Specification Revision History

June 12, 1989         Final revision approved by attendees of the VCPI

Developer's Conference, held at the offices of Phar

Lap Software on April 25, 1989.  Attendees included

the following VCPI sponsors:  A.I. Architects,

Inc., Lotus Development Corporation, Phar Lap

Software, Inc., Qualitas, Inc., Quarterdeck Office

Systems, Rational Systems, Inc.; and the following

observers:  Compaq Computer Corporation, Intel

Corporation.

May 8, 1989           Revision incorporating modifications agreed upon at

the VCPI Developer's Conference.  This revision was

distributed for approval to all conference

attendees.

March 31, 1989        A more detailed and explicit revision of the

December 1987 specification, drafted for review at

the VCPI Developer's Conference.

December 21, 1987     First revision of the specification, written by

Phar Lap Software, Inc., and Quarterdeck Office

Systems.

1.0  Background

The Intel 80386 microprocessor has three fundamental operating modes.  Real

mode is provided for backward compatibility with existing 8086 programs.

Protected mode allows programs written specifically for the 80386 to take

advantage of the larger address space available.  Virtual 8086 (V86) mode,

like real mode, is used to run 8086 programs.  However, V86 mode runs under

the control of a protected mode operating environment.  This provides certain

advantages, chiefly the ability to enable the paging hardware of the 80386 and

thus run multiple 8086 programs simultaneously, and also the ability to make

arbitrary physical memory available within the V86 address space of an 8086

program.  (The 80486 microprocessor provides the same architecture and

operating modes as the 80386, thus software written for the 80386 runs without

modification on an 80486).

The capabilities of the 8086 have spawned the creation of several new kinds of

control programs that can run under MS-DOS on a 386 machine.  To date, these

programs fall into three basic categories:  (1) protected mode run-time

environments, which allow an application program to execute in protected mode

under MS-DOS;  (2) EMS emulators, which use V86 mode to make all the memory on

the machine available to 8086 programs which use EMS (Lotus/Intel/Microsoft

Expanded Memory Specification) memory, and, (3) multitasking environments

which use V86 mode to multitask 8086 MS-DOS programs, while still giving each

8086 program a full 640Kb of physical memory (or more, if they use EMS

memory).  Such multitasking environments typically run in conjunction with a

separate EMS emulator, or implement the EMS interface as part of the

multitasker.  For the remainder of this specification, the terms "DOS-

Extender," "EMS emulator," and "multitasker," respectively, will be used to

refer to these program categories.

Since these control programs run under MS-DOS, it is desirable to make them

compatible with each other, so that users don't have to turn off one control

program in order to run an application under another control program.  The

purpose of this document is to specify an interface that allows these classes

of control programs to coexist successfully.  This interface is called the

Virtual Control Program Interface, or VCPI.

2.0  Areas of Conflict

There are a variety of resource contention issues which must be solved to

allow multiple 386 control programs to coexist.  The section below summarizes

each of these issues.

2.1  Extended Memory Allocation

Because DOS only knows about memory below one MB, there is no universally used

method in the DOS environment for allocating and freeing extended memory

(memory above one MB).  The following existing techniques are available for

allocating extended memory on both 80286 and 80386 PCs:

1. Memory can be allocated "top down" by taking over the BIOS extended

memory size system call (INT 15h function 88h) and reporting less

extended memory available than is actually present on the machine.

This method allocates a contiguous block of memory starting at the top

of extended memory and growing downward.  See Appendix A.1 for a

description of top down allocation.

2. Memory can be allocated "bottom-up" by taking over the PC reboot

interrupt (INT 19h) and installing a signature block and allocation

size marker in the interrupt handler, and boot block with an

______________________________________________________________________________

VCPI Version 1.0           June 12, 1989                      Page 1

allocation size marker at one megabyte.  This is commonly referred to

as the VDISK technique, since it is the method employed by the

original IBM VDISK driver.  This method allocates a contiguous block

of memory starting at one megabyte and growing upward.  See Appendix

A.2 for a description of bottom-up allocation.

3. If an XMS (Microsoft Extended Memory Specification) drive is

installed, extended memory can be allocated by making calls to the XMS

driver.  The XMS driver reserves extended memory for an XMS memory

pool using top-down allocation, and the allocates memory out of the

XMS memory pool.  Copies of the XMS specification may be obtained by

calling Microsoft at 1-800-426-9400.

A program that wishes to allocate extended memory must first determine what

extended memory is free (not in use by another program).  It does this by

calling INT 15h function 88h to obtain the top of free extended memory, and by

checking the signature of INT 19h and the boot block at one megabyte for any

bottom-up memory allocation, thus obtaining the bottom of free extended

memory.

The program can then allocate its own chunk of extended memory, either top

down by taking over INT 15h, or bottom-up by taking over INT 19h.  The top-down

allocation method is preferable because it is simpler and more robust.  The

diagram below shows an example in which three programs which use extended

memory are installed.  Program 1 (the first program installed) allocates some

extended memory bottom-up, program 2 allocates two chunks of memory, one

bottom-up and one top-down, and program 3 allocates memory top-down.

top of    _______________________________

memory    |XXXXXXXXXXXXXXXXXXXXXXXXXXX|<=  allocated by program 2

|XXXXXXXXXXXXXXXXXXXXXXXXXXX|

|---------------------------|--

|***************************|<=  allocated by program 3

|---------------------------|--

|                           |<=  free extended memory

|---------------------------|--

|XXXXXXXXXXXXXXXXXXXXXXXXXXX|<=  allocated by program 2

|XXXXXXXXXXXXXXXXXXXXXXXXXXX|

|---------------------------|--

|^^^^^^^^^^^^^^^^^^^^^^^^^^^|<=  allocated by program 1

1 MB   => |---------------------------|--

|///////////////////////////|<=  BIOS ROM, screen, etc.

640 KB => |---------------------------|--

|\\\\\\\\\\\\\\\\\\\\\\\\\\\|<=  DOS memory

|___________________________|__

Note that only the last installed program which allocates extended memory top-

down or bottom-up (the first program in the interrupt chain for INT 15h or INT

19h) can dynamically change its memory allocation. In the example above, only

program 2 can dynamically modify its bottom-up memory allocation, and only

program 3 can dynamically modify its top-down memory allocation.

______________________________________________________________________________

VCPI Version 1.0           June 12, 1989                      Page 2

It is possible for VCPI memory, XMS memory and extended memory all to be

available on a single PC.  (This can occur, for example, if a VCPI driver is

installed and is given a portion of extended memory, an XMS driver is

installed and even another chunk of extended memory, and the rest of extended

memory is left free.)  Therefore, programs which can use extended memory may

wish to look for all three sources of memory.  The recommended order of use of

the different memory allocation methods is:  (1) VCPI memory, (2) XMS memory,

and, (3) extended memory.

2.2  Mode Switching

All 386 control programs need to do some processing in protected mode, but

also need to allow standard 8086 code (such as other applications, DOS, and

the BIOS) to execute.  Therefore, they must switch between 80386 protected mode

and either real mode or V86 mode.

If two control programs both switch between protected mode and real mode, no

special problems are posed.  If, however, a control program switches between

V86 mode and protected mode, it is impossible for a second control program to

switch into protected mode.  This is because programs running in V86 mode are

executing at the least privileged level of the 80386 processor, so system-

level operations will cause processor exceptions, which result in the first

control program regaining control.  Thus the first control program must

provide a method to allow subsequent control programs to switch to protected

mode and back to V86 mode again.

2.3  8259A Interrupt Controller

The standard interrupt vectors used on the 386 PC for hardware interrupts

IRQ0 - IRQ7 are vectors 08h - 0Fh.  Unfortunately, those vectors are also used

by the 80386 processor for certain processor exceptions.  For this reason,

some 386 control programs reprogram the 8259A interrupt controller chip to

relocate IRQ0 - IRQ7 to different interrupt vectors.  Information about

reprogramming of the 8259A must be made available to all cooperating 386

control programs.

2.4  Privileged Instructions

When the 386 processor is executing in V86 mode, it is executing at privilege

level 3 (the least privileged level), and privileged instructions cause an

exception.  These include instructions which load or store 386 system

registers, including the control registers, the debug registers, the test

registers, GDTR, LDTR, and TR.  Access to some of these registers is needed in

V86 mode; others only need to be modified as part of a mode switch to

protected mode.

In addition, a number of instructions cause a general protection exception if

the I/O privilege level (IOPL) is not 3 when executing in V86 mode.  These

instructions must be made available, either by setting IOPL to 3, or by

emulating the instructions.

3.0  Interface Summary

Many of the problems listed above occur only when one control program uses V86

mode.  EMS emulators run in V86 mode and are widely used on 386 PCs, and an

EMS interface is typically used or provided by multitaskers. In addition, one

of the primary functions performed by EMS emulators is memory allocation.

______________________________________________________________________________

VCPI Version 1.0           June 12, 1989                      Page 3

For these reasons, the VCPI interface is defined as a superset of the EMS

version 4.0 interface (an EMS 4.0 driver must be provided underneath the VCPI

interface; a copy of the EMS 4.0 interface specification may be obtained by

calling Intel literature sales at 1-800-548-4725 and requesting part #300275).

If an EMS emulator with the VCPI interface is installed, a multitasker and/or

an arbitrary number of DOS-Extender programs which use the VCPI interface can

be run.  Control programs which utilize the VCPI interface provided by the EMS

emulator will be referred to as "clients" for the remainder of this document.

The control program providing the VCPI interface (and the EMS 4.0 interface)

will be referred to as the "server" for the remainder of this document.

A server is installed at boot time, and takes over a block of extended memory

which becomes the expanded memory pool from which it allocates EMS pages.

When the server is installed, it can be given all the extended memory on the

computer, or extended memory can be "partitioned" into expanded memory and

extended memory (i.e., some extended memory is taken by the server for the

expanded memory pool, and the rest is left free, available for use by other

programs).

Clients can obtain memory by making requests of the server.  Memory allocated

though the VCPI interface comes out of the memory pool owned by the server

(i.e., expanded memory), and must be in the physical address space of the

computer (e.g., it must not be memory on a separate expanded memory board).

If any extended memory is left free by the server, clients can allocate that

extended memory in the usual way.

The VCPI interface is implemented with calls to functions provided by the

server.  Calls need to be made both in V86 mode and in protected mode.  The

V86 mode calls are implemented through the standard INT 67h EMS interface.

The protected mode calls are made with FAR procedure calls to an entry point

in the server.  The protected mode entry point is obtained via an

initialization on call to the server.

Both the server and each client program maintain their own set of system

tables (GDT, LDT, IDT, page tables, etc.)  Each client program must initialize

a memory space in its page tables which is identical to a portion of the

memory space used by the server, in order to provide a common ground for

communication with the server.  When the client calls the server to switch to

protected mode, the client's system tables are set up as part of the mode

switch.  Likewise, when the client calls the server to switch back to V86

mode, the server's system tables are set up.

The server must leave A20 (address line 20 on a PC) enabled at all times, in

both V86 mode and in protected mode.

The client must always use the services of the server to switch modes. It

should never, for example, implement its own switch back to V86 mode, because

the server's system tables must be set up when executing in V86 mode.

4.0  V86 Mode Program Interface

The V86 mode interface is provided through the EMS interrupt (INT 67h), with

an EMS function code in AH of DEh, and a VCPI function code in AL. (Note that

any EMS driver which does not provide VCPI should return either code 84h (bad

function code) for this EMS call, since it is not a defined function number

for the LIM 4.0 EMS specification.)  If the function succeeds, AH is returned

as zero; otherwise, there is an error code in AH. Except as noted for each

call, all other registers are unchanged on output.

_______________________________________________________________________________

VCPI Version 1.0           June 12, 1989                      Page 4

If an undefined VCPI subfunction code is passed in AL, the server returns an

either code in AH; the recommended value is 8Fh (bad subfunction code).

The IOPL-sensitive privileged instructions must be available in V86 mode.

This means the server must either set the IOPL to 3, or emulate these

instructions.  Other privileged instructions need not be emulated as part of

the VCPI interface.  Instead, access to certain system registers is provided

via system calls (see Section 4.4).

4.1  Presence Detection

The presence of a server which provides the VCPI interface is detected by

first using one of the standard detection checks to see if an EMS driver is

installed, and then making the EMS call below to check for the presence of

VCPI.

==============================================================================

INT 67h

AX = DE00h     VCPI Presence Detection

==============================================================================

INPUTS:        None

-------------------------------------------------------------------------------

OUTPUTS:       If VCPI is present:

AH = 0

BL = 0 (VCPI minor version number, in binary)

BH = 1 (VCPI major version number, in binary)

If not present:

AH = nonzero (recommended value:  84h)

-------------------------------------------------------------------------------

If the server is installed but no EMS memory has been allocated, the server

may be executing in real mode rather than V86 mode.  The detection call should

return success in real mode as well as V86 mode (so that it is possible to

detect a VCPI server that is currently turned off, and also for backward

compatibility with clients that make this call in real mode); however, for the

rest of the VCPI interface to be enabled, the processor must be executing in

V86 mode.  Therefore, the following VCPI detection sequence is recommended:

1.  Check to see if the processor is an 80386 or 80486.

2.  Check to see if an EMS driver is installed.

3.  Allocate one EMS page.  This will turn on the EMS driver if it is

off, switching the processor to V86 mode.

4.  Make the above VCPI detection call.

The EMS page allocated in step 3 must not be deallocated again until the

client is done using the VCPI interface; otherwise, the server could turn off

again, setting the processor back to real mode and disabling the VCPI

interface.

Appendix A.3 contains a code sequence demonstrating VCPI detection.

4.2  Interface Initialization

The interface initialization process involves creating a common ground between

the server and the client that allows them to communicate in protected mode,

and providing any other information that the client may need to complete its

______________________________________________________________________________

VCPI Version 1.0           June 12, 1989                      Page 5

own initialization.  Since the server and the client each have separate page

tables, GDT, LDT, and IDT, in order to communicate in protected mode the

client must set up a region in its page tables that maps memory used by the

server, and must initialize segment descriptors in its GDT to point to code

and data regions in the server.

The calls below are used to initialize the interface between the server and

the client.

==============================================================================

INT 67h

AX = DE01h     Get Protected Mode Interface

==============================================================================

INPUTS:        ES:DI = pointer to 4K page table buffer

DS:SI = pointer to three descriptor table entries in the

client's GDT, the first to be set up as the

protected mode code segment in the server, and the

additional two entries to be used as desired by

the server when it is called in protected mode

-------------------------------------------------------------------------------

OUTPUTS:       AH = 0

DI = advanced to point to the first unused page table

entry in the buffer (could be advanced by 4K, if

server initializes entire page table)

EBX = offset in code segment of protected mode entry point

-------------------------------------------------------------------------------

The Get Protected Mode Interface call is used to set up a common frame of

reference which allows the client to make calls to the server in protected

mode.  The call initializes a page table which the client must set up as its

0th page table.  This page table must map at least one megabyte of memory

starting at linear address zero:  the real mode address space provided in V86

mode.  In addition, the server can map any additional memory above one

megabyte (up to a total of four megabytes, since there is just one page table)

which it requires to provide the protected mode interface.  While the mappings

for the first megabyte must be identical for both the client and the server

(so the client will be able to set up segments to access its V86 memory from

protected mode), memory above one megabyte can be mapped as desired by the

server (e.g., if the server's code and data is mapped at a high address in its

own page tables, it would simply map the same code and data just above one

megabyte in the client's 0th page table).

For backward compatibility, both the server and the client must clear the

software-defined bits (bits 9 - 11) in the page table entries it copies into

the client's page table; all other bits should be preserved the client must

not copy its 0th page table to a different physical page after this call is

made.  This is so the server can always access the client's 0th page table.

The client must never modify any bits other than the software-defined bits (9-

11) in the page table entries which are initialized by this call.

In addition to initializing the 0th page table, this call also fills in a GDT

entry in the client's GDT.  This segment is used by the client when calling

the server in protected mode.  The segment must be set up as a code segment,

and must reside in the linear address space set up in the 0th page table by

this call.  The entry point of the interface routine (its offset within the

code segment) is returned in EBX.  Two additional entries in the client's GDT,

immediately following the GDT entry for the code segment, are available to the

server for its own use.  Typically at least one of these GDT segments is used

by the server as a data segment.

______________________________________________________________________________

VCPI Version 1.0           June 12, 1989                      Page 6

When the client calls the server in protected mode, it makes a FAR call to a

USE32 segment, using the entry point provided by this call.  Since the server

has initialized the code segment descriptor, it can, of course, set it up any

way it desires, but it is responsible for executing a USE32 FAR return when it

terminates, regardless of the actual use type of the code segment.  Thus, when

the server is called in protected mode, it will have CS pointing to the code

segment which is set up by this call.  It can obtain selectors for the two

additional GDT entries which it set up by adding 0008h and 0010h,

respectively, to the segment selector value in CS.  The server must preserve

all segment registers across a protected mode VCPI call.

==============================================================================

INT 67h

AX = DE02h     Get Maximum Physical Memory Address

==============================================================================

INPUTS:        None

------------------------------------------------------------------------------

OUTPUTS:       AH = 0

EDX = PHYSICAL address of the highest 4K memory page that

could ever be allocated.

------------------------------------------------------------------------------

The Get Maximum Physical Memory Address call is used by the client to

initialize its memory management data structures.  To do this, it may need to

know the physical address of the highest 4K memory page that could ever be

allocated by the Allocate 4K Page call specified below.  For backward

compatibility, both the server and the client must mask the 12 LSBs of the

physical page address (returned in EDX) to zero.

4.3  Memory Allocation Calls

The memory allocation calls are used to allocate and free 4K pages of memory.

The server allocates these pages out of its EMS memory pool.  The client can

also do its own allocation of extended memory or XMS memory, if any is

available.

==============================================================================

INT 67h

AX = DE03h     Get Number of Free 4K Pages

===============================================================================

INPUTS:        None

-------------------------------------------------------------------------------

OUTPUTS:       AH = 0

EDX = number of free 4K pages

-------------------------------------------------------------------------------

This call returns the total number of 4K pages available to be allocated out

of the server's expanded memory pool.

Note that this is the total amount of free memory available to all tasks on

the system.  The client should allocate this memory only on an as needed

basis. In addition, in a multitasking environment, this number does not

reflect any memory restrictions placed on the client task by the multitasker.

The client should, during initialization, use the EMS call Get Number of Pages

(INT 67h function 3 (AH=42h)) to obtain the number of unallocated EMS pages

available, and then limit its usage of memory allocated through the server to

that amount.  (Note that the size of an EMS page is 16K, and the size of a

page allocated through the VCPI interface is 4K.)

______________________________________________________________________________

VCPI Version 1.0           June 12, 1989                      Page 7

==============================================================================

INT 67h

AX = DE04h     Allocate a 4K Page

===============================================================================

INPUTS:        None

-------------------------------------------------------------------------------

OUTPUTS:       If success:

AH = 0

EDX = PHYSICAL address of allocated 4K page

If failure:

AH = nonzero (recommended value:  88h)

EDX = contents modified

-------------------------------------------------------------------------------

This call allocates a 4K page of memory.  For backward compatibility, both the

server and client must make the 12 LSBs of the physical page address (returned

in EDX) to zero.  The client is responsible for freeing all memory allocated

with this call before terminating execution.

==============================================================================

INT 67h

AX = DE05h     Free a 4K Page

===============================================================================

INPUTS:        EDX = PHYSICAL address of 4K page to free

-------------------------------------------------------------------------------

OUTPUTS:       If success:

AH = 0

If failure:

AH = nonzero (recommended value:  8Ah)

-------------------------------------------------------------------------------

This call frees a page of memory previously allocated with subfunction 04h.

For backward compatibility, both the server and the client must mask the 12

LSBs of the physical page address (passed in EDX) to zero.

______________________________________________________________________________

VCPI Version 1.0           June 12, 1989                      Page 8

==============================================================================

INT 67h

AX = DE06h     Get Physical Address of 4K Page in First Megabyte

===============================================================================

INPUTS:        CX = page number (linear address of page SHR 12)

-------------------------------------------------------------------------------

OUTPUTS:       If success:

AH = 0

EDX = PHYSICAL address of 4K page

If invalid page number:

AH = nonzero (recommended value:  8Bh)

-------------------------------------------------------------------------------

This call returns the physical address of a 4K page in the first megabyte of

the V86 mode linear address space.  For backward compatibility, both the

server and the client must mask the 12 LSBs of the physical page address

(returned in EDX) to zero.

This call is useful for obtaining the address of memory which is allocated and

mapped into the first megabyte with the standard EMS calls.  While this

information can also be extracted from the page table obtained with the Get

Protected Mode Interface system call, that page table is a "snapshot" of the

server's page tables at the time the system call was made, and would not have

the correct addresses for any EMS memory mapped in after the Get Protected

Mode Interface call was made.

4.4  System Register Access Calls

When executing in V86 mode, any attempt to load or store system registers

causes a general protection exception.  The system calls below provide access

to certain system registers in V86 mode.

===============================================================================

INT 67h

AX = DE07h     Read CR0

===============================================================================

INPUTS:        None

-------------------------------------------------------------------------------

OUTPUTS:       AH = 0

EBX = CR0 value

-------------------------------------------------------------------------------

This call returns the current value of the CR0 register.

______________________________________________________________________________

VCPI Version 1.0           June 12, 1989                      Page 9

===============================================================================

INT 67h

AX = DE08h     Read Debug Registers

===============================================================================

INPUTS:        ES:DI = pointer to array of 8 DWORDS, DR0 first in

array, DR4 and DR5 not used

-------------------------------------------------------------------------------

OUTPUTS:       AH = 0

-------------------------------------------------------------------------------

This call stores the values of the debug registers into an array in memory.

===============================================================================

INT 67h

AX = DE09h     Load Debug Registers

===============================================================================

INPUTS:        ES:DI = pointer to array of 8 DWORDS, DR0 first in array,

DR4 and DR5 not used

-------------------------------------------------------------------------------

OUTPUTS:       AH = 0

-------------------------------------------------------------------------------

This function loads the debug registers with the values in the array.

4.5  8259A Interrupt Vector Mapping Calls

The default PC interrupt vector assignments for hardware interrupts are

vectors 08h - 0Fh for interrupts IRQ0 - IRQ7, and vectors 70h - 77h for

interrupts IRQ8 - IRQ15.  The 80386 processor uses interrupt vectors 08h - 0Fh

for processor exceptions, which are important to 386-specific software.

Therefore, some clients find it desirable to reprogram the 8259A interrupt

controller to map the hardware interrupt vectors elsewhere in the interrupt

space.  The server, as the first control program in place, is the repository

of information on what the current vector mappings are.  Client programs

should query the server during initialization to determine the current

mappings. If they have been changed from the PC default mappings, the client

is obligated not to change them again.  If they are still set to the PC

defaults, the client can reprogram the interrupt controller and then inform

the server what the new mappings are.

______________________________________________________________________________

VCPI Version 1.0           June 12, 1989                      Page 10

===============================================================================

INT 67h

AX = DE0Ah     Get 8259A Interrupt Vector Mappings

===============================================================================

INPUTS:        None

-------------------------------------------------------------------------------

OUTPUTS:       AH = 0

BX = 1st vector mapping for master 8259A (IRQ0-IRQ7)

CX = 1st vector mapping for slave 8259A (IRQ8-IRQ15)

-------------------------------------------------------------------------------

This function returns the interrupt vectors generated by the 8259A interrupt

controller chips when a hardware interrupt occurs.  If there is no slave 8259A

to generate IRQ8-IRQ15 (because the software is running on a 386 accelerator

card plugged into a PC or a PC/XT), then the value returned in CX is

unspecified.

Note that, for programs who wish to take over hardware interrupts in V86 mode,

other considerations may apply when executing in a multitasking environment.

===============================================================================

INT 67h

AX = DE0Bh     Set 8259A Interrupt Vector Mappings

===============================================================================

INPUTS:        BX = vector mapping for master 8259A (IRQ0-IRQ7)

CX = vector mapping for slave 8259A (IRQ8-IRQ15)

Interrupts disabled

-------------------------------------------------------------------------------

OUTPUTS:       AH = 0

-------------------------------------------------------------------------------

This call is used by the client to inform the server if it remaps the 8259A

interrupt controllers.  The client must leave interrupts disabled from the

time it begins to reprogram the interrupt controllers until after it makes

this function call to inform the server of the remapping.

Note that the server must leave interrupts disabled until after it has logged

the vector mappings provided by the call.  In particular, it must not

"reflect" the interrupt back to V86 mode before recording the information, in

case there is another program in the INT 67h chain which enables interrupts.

If the client remaps the 8259A interrupt controller, it is responsible for

restoring the original interrupt controller mappings when it terminates.

______________________________________________________________________________

VCPI Version 1.0           June 12, 1989                      Page 11

4.6  Switch to Protected Mode

===============================================================================

INT 67h

AX = DE0Ch     Switch From V86 Mode to Protected Mode

===============================================================================

INPUTS:        ESI = linear address of data structure in first megabyte,

containing values to load into system registers

Interrupts disabled

-------------------------------------------------------------------------------

OUTPUTS:       GDTR, IDTR, LDTR, TR loaded

Interrupts disabled

Control transferred to specified FAR entry point

SS:ESP must have at least 16 BYTEs of space on it, and the

protected mode entry point is required to set up its

own stack before re-enabling interrupts

EAX = contents modified

ESI = contents modified

DS, ES, FS, GS = contents modified

All other registers unmodified

-------------------------------------------------------------------------------

This call switches to protected mode, sets up all the system tables for the

client, and transfers control to the specified entry point in the client.

Interrupts must remain disabled for the entire duration of the switch.  The

ESI register contains a linear address of a data structure in the first

megabyte; because the page table mappings for the first megabyte are identical

in both the server's and the client's page tables, this address can be used by

the server both before and after reloading the CR3 register.

The data structure pointed to by ESI looks like:

10 (DWORD) CS:EIP too transfer control to

0E (WORD)  selector value to load into TR

0C (WORD)  selector value to load into LDTR

08 (DWORD) linear address in first megabyte of 6-byte value to

load into IDTR

04 (DWORD) linear address in first megabyte of 6-byte value to

load into GDTR

ESI->00 (DWORD) new value to load into CR3

Note that the server must load GDTR before loading the LDTR and TR (IDTR can

be loaded at any time, since interrupts are disabled).  The GDTR and IDTR must

be loaded with the 32-bit forms of the LGDT and LIDT instructions, not the 24-

bit forms provided for backward compatibility with the 80286 processor.  The

server must clear the task busy bit in the TSS descriptor table entry in the

client's GDT before loading TR (which must be loaded with an LTR instruction,

not a task switch).  Note that, since the client's GDT could reside anywhere

in the 4 GB linear address space, the TSS descriptor table entry cannot be

modified (and the LDTR and TR registers cannot be loaded) until after CR3 is

loaded with the client's value, and the data segment used to modify the TSS

descriptor must have a base address of zero and a limit of 4 GB.  The server

must not reload any segment registers after it loads the GDTR with the

client's GDT address; note that the ability to modify GDTR and LDTR without

reloading segment registers relies on the segment descriptor caching performed

by the processor.

______________________________________________________________________________

VCPI Version 1.0           June 12, 1989                      Page 12

The server's code that implements the switch must reside in the linear region

that was initialized with the Get Protected Mode Interface call (function

DE01h), since that is the only linear region that is mapped identically in

both the server's and the client's page tables.

When the client gets control, the stack is still set to whatever stack the

server was using (because SS has not been reloaded, the descriptor cache

register for SS still points to the server's stack segment).  The client must

reload all segment registers and set up its own stack before enabling

interrupts.  This is a requirement since an interrupt handler will attempt to

save and restore segment registers, and the values left in the segment

registers after the switch are not necessarily valid segments for the client.

5.0  Protected Mode Interface

The protected mode interface is a FAR entry point in the server, which is

obtained during initialization with the Get Protected Mode Interface system

call (INT 67h function DE01h).  The client makes a FAR call to a USE32 segment

when it calls the server entry point, and the server must execute a USE32 FAR

return when it terminates.

An EMS function code of DEh is passed in AH, and a VCPI function code is

passed in AL.  If the function succeeds, AH is returned as zero; otherwise,

there is an error code in AH.  Except as noted for each call, all other

registers are unchanged on output.

If an undefined VCPI subfunction code is passed in AL, the server returns an

error code in AH; the recommended value is 8Fh (bad subfunction code).

5.1  Memory Allocation Calls

The memory allocation calls are used to allocate and free 4K pages of memory

the server allocates these pages out of its expanded memory pool.  The client

can also do its own allocation of extended memory or XMS memory, if any is

available.

===============================================================================

INT 67h

AX = DE03h     Get Number of Free 4K Pages

===============================================================================

INPUTS:        None

-------------------------------------------------------------------------------

OUTPUTS:       AH = 0

EDX = number of free 4K pages

-------------------------------------------------------------------------------

This call returns the total number of 4K pages available to be allocated out

of the server's expanded memory pool.

Note that this is the total amount of free memory available to all tasks on

the system.  The client should allocate this memory only on an as needed

basis.  In addition, in a multitasking environment, this number does not

reflect any memory restrictions placed on the client task by the multitasker.

The client should, during initialization, use the EMS call Get Number of Pages

(INT 67h function 3 (AH = 42h)) to obtain the number of unallocated EMS pages

available, and then limit its usage of memory allocated through the server to

that amount.  (Note that the size of an EMS page is 16K, and the size of a

page allocated through the VCPI interface is 4K.)

______________________________________________________________________________

VCPI Version 1.0           June 12, 1989                      Page 13

===============================================================================

INT 67h

AX = DE04h     Allocate a 4K page

===============================================================================

INPUTS:        None

-------------------------------------------------------------------------------

OUTPUTS:       If success:

AH = 0

EDX = PHYSICAL address of allocated 4K page

If failure:

AH = nonzero (recommended value:  88h)

EDX = contents modified

-------------------------------------------------------------------------------

This call allocates a 4K page of memory.  For backward compatibility, both the

server and client must mask the 12 LSBs of the physical page address (returned

in EDX) to zero.  The client is responsible for freeing all memory allocated

with this call before terminating execution.

===============================================================================

INT 67h

AX = DE05h     Free a 4K Page

===============================================================================

INPUTS:        EDX = PHYSICAL address of allocated 4K page

-------------------------------------------------------------------------------

OUTPUTS:       If success:

AH = 0

If failure:

AH = nonzero (recommended value:  8Ah)

-------------------------------------------------------------------------------

This call frees a page of memory previously allocated with subfunction 04h.

For backward compatibility, both the server and the client must mask the 12

LSBs of the physical page address (passed in EDX) to zero.

______________________________________________________________________________

VCPI Version 1.0           June 12, 1989                      Page 14

5.2  Switch to V86 Mode

===============================================================================

INT 67h

AX = DE0Ch     Switch From Protected Mode to V86 Mode

===============================================================================

INPUTS:        Top of stack looks as described below

Interrupts disabled

SS:ESP must be in the first megabyte of linear memory

DS = segment selector that maps the entire linear address space

that was obtained with subfunction 01h, Get Protected Mode

Interface (i.e., its base address is 0, and its limit is

the size of the address space obtained with the Get

Protected Mode Interface call)

-------------------------------------------------------------------------------

OUTPUTS:       GDTR, IDTR, LDTR, TR loaded with server's values

Interrupts disabled

Control transferred to specified FAR entry point

SS:ESP and all segment registers loaded with values on stack

EAX = destroyed

All other registers unmodified

-------------------------------------------------------------------------------

This call switches from protected mode back to V86 mode, after setting up all

of the server's system tables.  The top of stack looks as follows on input to

this function:

28 (DWORD) GS value

24 (DWORD) FS value

20 (DWORD) DS value

1C (DWORD) ES value

18 (DWORD) SS value

14 (DWORD) ESP value

10 (DWORD) reserved for EFLAGS value

0C (DWORD) CS value

08 (DWORD) EIP value

SS:ESP-> 00 (DWORD) return address from FAR call to USE32 segment

The first 8 bytes at SS:ESP are the return address pushed by the FAR CAlL

instruction; since this call never returns, the return address is irrelevant

and should be discarded.  It is the responsibility of the server to fill in an

appropriate EFLAGS value (e.g., VM bit set, IOPL = 3, IF bit cleared) before

executing the IRETD to return to V86 mode.  Interrupts must remain disabled

for the entire duration of the switch, and the interrupt enable bit must be

off in the EFLAGS value which is stored on the stack before executing the

IRETD which switches back to V86 mode.  The server's code that implements the

switch must reside in the linear region that was initialized with the Get

Protected Mode Interface call (function DE01h), since that is the only linear

region that is mapped identically in both the server's and the client's page

tables.

If the client does any task switching, it should clear the TS bit in CR0

before making this call.  Otherwise, the TS bit will still be set after the

switch to V86 mode, and a floating point coprocessor instruction in V86 mode

will cause an exception 7 trap to the server.

______________________________________________________________________________

VCPI Version 1.0           June 12, 1989                      Page 15

Appendix A

Implementation Notes

A.1  Top-Down Extended Memory Allocation

The BIOS extended memory size system call is used to determine the upper limit

of extended memory which is not in use by other programs.  The following code

fragment gets the physical address of one byte past the end of available

extended memory in the EAX register:

xor  eax,eax             ; clear MSBs in EAX

mov  ah, 88h             ; get size of extended memory in KB

int  15h

shl  eax, 10             ; convert it address of top of memory

add  eax, 100000h

To allocate extended memory top down, simply install an INT 15h handler which

passes all functions except 88h through to the previous INT 15h handler.  If

an INT 15h function 88h call is made, return a value which reduces the size of

extended memory by the amount of memory you wish to allocate.

A.2  Bottom-Up Extended Memory Allocation

Bottom-up extended memory allocation is not recommended, as it is more

complicated and less versatile than top-down allocation.  However, programs

that wish to allocate extended memory must first check to see if any other

programs have allocated extended memory using the bottom-up technique.  Bottom-

up extended memory allocation is done by marking the amount of memory

allocated in two locations.  If there is any extended memory on the system (if

the BIOS call documented in Appendix A.1 does not return zero), it is

necessary to check BOTH of these locations to see if any bottom-up extended

memory has been allocated.  If the two locations show different amounts of

allocated memory, the safest thing to do is terminate with an error message.

Failing that, the larger of the two values found should be used.  The values

may differ in the two locations because there are several programs available

which use bottom-up allocation but which do not correctly update both

locations, and also because the DOS PRINT command in DOS 3.3 or later wipes

out the allocation information in one of the two locations.

The first location is the segment pointed at by the INT 19h vector (the reboot

interrupt).  If an appropriate signature is present at a specific offset in

this segment, then there is a 24-bit address of the first free byte of

extended memory at another specific offset in the segment.  The following code

fragment obtains the physical address of the first free byte of extended

memory and loads it into EAX:

data segment

vdisk_sig db 'VDISK V'       ; signature string

SIG _ LEN equ $ - vdisk_sig  ; signature string length

SIG_VEC equ 19h              ; interrupt vector for signature

SIG_OFFS equ 12h             ; offset of signature string in segment

LO_OFFS equ 2Ch              ; offset of low word of 24-bit value

HI_OFFS equ 2Eh              ; offset of high byte of 24-bit value

data ends

______________________________________________________________________________

VCPI Version 1.0           June 12, 1989                      Page 16

assume    cs:code,ds:data

code      segment use16

xor ax,ax                 ; get segment addr in ES

mov es,ax

mov ax,es:(SIG_VEC * 4) + 2

mov es,ax

mov di,SIG_OFFS           ; branch of signature not present

lea si,vdisk_sig

mov ex,SIG_LEN

cld

repe cmpsb

jne short no_alloc

xor eax,eax               ; get addr of bottom of extended memory

mov al,es:HI_OFFS         ; into EAX

shl eax,16

mov ax,es:LO_OFFS

jmp short done            ; all done

no_alloc:

mov eax,100000h      ; no bottom-up memory allocated

done:                          ; bottom of extended memory is now in EAX

code ends

The second location to check is a boot block located at one megabyte.  The

following C code fragment retrieves the allocation value from the boot block

and saves it in the variable "bot_ext" (note that this code should NOT be

executed if there is no extended memory on the system, since it attempts to

read memory at one megabyte):

long bot_ext;        /* addr of 1st free byte of extended memory */

char buf [0x20];     /* buffers boot block */

#define SIGOFF 0x03  /* offset of signature in boot block */

#define SIZEOFF 0x1E /* offset of size word in boot block */

/*

* Read boot block at one megabyte by calling an assembly language

* routine (not shown) which either uses the BIOS block move call to

* read from extended memory, or does it in some other way.

*/

read_ext(0x100000, buf, sizeof(buf));

/*

* Now check for the correct signature in the boot block, and if

* present, extract the bottom of available extended memory.

*/

if (memcmp(&buf[SIGOFF], "VDISK", 5) != 0)

bot_ext = 0x100000;

else

{

bot_ext = 01;

memcpy(&bot_ext, &buf[SIZEOFF], 2]; /* get KB addr */

bot_ext = bot_ext << 10;            /* convert to byte addr */

______________________________________________________________________________

VCPI Version 1.0           June 12, 1989                      Page 17

A.3  Sample VCPI Detection Code

The following code sequence should be used to detect the presence of the VCPI

interface:

data segment

emm_name db 'EMMXXXX0',0      ; device name for EMM

data ends

assume      cs:code,ds:data

code segment use16

;

; First check to make sure we are on an 80386 or 80486

;

pushf               ; save flags

xor ax,ax           ; try to put a zero word into flags and then

push ax             ; get result back into AX

popf

pushf

pop ax

and ax,0F000h       ; if high 4 bits of flags are all 1's, it's an

cmp ax,0F000h       ; 8086/8088/80186/80188

jne short not_386

mov ax,0F000h       ; Now try to set the high 4 bits of the flags

push ax             ; and then get the result back in AX

popf

pushf

pop ax

and ax,0F000h        ; if none of the high 4 bits are set, it's an

jz short not_386     ; 80286 -- otherwise, it's a 386 or 486

popf                 ; restore flags

jmp short ems_check  ; go to next check

not_386:

popf                 ; restore flags

jmp no_vcpi          ; not a 386 or 486 -- VCPI not present

ems_check:

;

; Check for EMS driver present

;

lea dx,emm_name      ; open EMM device, read-only

mov ax,3D00h

int 21h

jc short no_vcpi     ; branch if error

mov bx,ax            ; get device information

mov ax,4400h

int 21h

jc short no_vcpi     ; branch if error

test dx,80h          ; branch if file (not device)

jz short no_vcpi

mov ax,4407h         ; check output status

int 21h

push ax              ; save return code from IOCTL call

mov ah,3Eh           ; close file

int 21h

pop ax               ; restore IOCTL return code

cmp al,0FFh          ; branch if status not device ready

jne short no_vcpi

______________________________________________________________________________

VCPI Version 1.0           June 12, 1989                      Page 18

; Now allocate an EMS page to make sure the EMS emulator is turned on.

; Note that we must save the handle for this allocated page and free it

; again eventually, although this is not shown in the code fragment.

;

mov bx,1           ; allocate 1 EMS page

mov ah,43h

int 67h

cmp ah,0          ; branch if error

jne short no_vcpi

; save handle returned in DX to be freed later

; Now make VCPI detection call

;

mov ax,0DE00h        ; is VCPI there?

int 67h

cmp ah,0             ; branch if not there

jne short no_vcpi

; OK, the VCPI interface is present and the server is turned on.

no_vcpi:                 ; VCPI interface not present

code ends

______________________________________________________________________________

VCPI Version 1.0           June 12, 1989                      Page 19

GLOSSARY

4K page                  A four-kilobyte page, the page size allocated by

calls to the VCPI interface.  A 4K page is always

aligned on a four-kilobyte physical address boundary.

A20                      Address line 20.  On AT and MCA architectures, there

is hardware to truncate physical addresses to 20 bits

(one megabyte).  This is known as "disabling A20," and

is for compatibility with 8086 programs which rely on

address wraparound at one megabyte.  To use extended

memory, it is necessary to "enable A20."

bottom-up allocation     Allocation of extended memory starting at one

megabyte and growing upward.  This allocation method

is used by older VDISK device drivers.

client                   A program which uses the VCPI interface

expanded memory          Memory allocated by an EMS emulator

extended memory          Physical memory above one megabyte.

EMS page                 A 16-kilobyte page, the page size allocated by calls

to an EMS driver.  An EMS page is always allocated on

a four-kilobyte physical address boundary.

real mode address space  Linear addresses below one megabyte (note that this

does not necessarily correspond to physical addresses

below one megabyte)

server                   A program which provides the VCPI interface

top-down allocation      Allocation of extended memory starting at the top of

memory and growing downward.  This is the recommended

technique for allocating extended memory.

______________________________________________________________________________

VCPI Version 1.0           June 12, 1989                      Page 20