WHATVGA version 1.0   23/jan/93

Copyright 1991,982,93   Finn Thgersen

Command line switches:

whatvga [+acumos] [-ncr] [=1024]

where +chipset  forces WHATVGA to proceed as if this chipset was found.

The test for this chip set will be executed in order

to check for memory and revision.

No other chipsets will be tested.

-chipset  ensures that the tests for this chipset will NOT be

executed. Useful if a chipset is misidentified

or some earlier test gives problems.

=kbytes   Forces the number of Kbytes of video RAM on the

adapter to the specified value.

Whatvga has 6 options in the main menu:

1 Test standard VGA

Allows you to test the standard VGA video modes.

Draws a test pattern, including a color palette

and writes basic information as text on the display.

2 Test extended modes

Allows you to test the extended video modes of the adapter.

Draws a test pattern, including a color palette

and writes basic information as text on the display.

On the upper and left edge every hundred pixels is marked.

3 Scroll test

Allows you to test the scroll function of the adapter.

Draws a test pattern, including a color palette

and writes basic information as text on the display.

The test pattern is drawn in the largest resolution there

is video memory for.

With the arrow keys you can move around in the pattern.

4 Search for video modes

Steps through the video modes 14h to 7Fh to see which ones

are implemented by this adapter.

When a video mode is found the rutine attemps to analyse

the mode and then draws a test pattern to check the analysis.

When the test pattern is displayed you can view the register

values by pressing the "d" key, or save them to a file named

REGISTER.VGA by pressing the "f" key.

5 Test VESA modes

This item is only shown if a VESA driver is found.

Draws a test pattern, including a color palette

and writes basic information as text on the display.

9 Exit

Adapters tested:

Ident:   Memory:   Ext res:   Scroll:

Acumos:

AVGA2          OK       Ok        Ok         No *

AVGA3

Ahead:

A

B

ATI:

18800

18800-1

28800-2

28800-4

28800-5

Avance Logic:

AL2101         Ok       Ok        Ok *       Ok *

Chips&Tech:

82c451

82c452

82c453

82c455

82c456

82c457

F65520

F65530

Cirrus:

510/520

610/620

5422           Ok       Ok        Ok         Ok

Compaq:

Everex:

Genoa:

5100/5200

5300/5400

6100

6200/6300

6400/6600

7900

MXIC:

86010          Ok       Ok        Ok

NCR:

77C20

77C21

77C22E         Ok       Ok        Ok         Ok *

77C22E+

Oak:

OTI-37C        Ok       Ok

OTI-067        Ok       Ok        Ok

OTI-077

Paradise/Western Digital:

PVGA1A         Ok       Ok        Ok         Ok

WD90C00

WD90C10

WD90C11

WD90C2x

WD90C30        Ok       Ok        Ok         Ok

WD90C31

Primus:

P2000          Ok       Ok        Ok         Ok

Realtek:

RT3106         Ok       Ok        *          *

S3:

86c911

86c911A

86c924         *        Ok        Ok         Ok

86c801

86c805

86c928

Trident:

8800BR

8800CS

8900

8900C

9000

Tseng:

ET3000         Ok       Ok        Ok         *

ET4000         Ok       Ok        Ok         Ok

ET4000W32

Video7/Headland:

HT-208

HT-216

Yamaha:

6388

Acumos AVGA2:

Scroll doesn't work.

Avance Logic:

Scroll has problems.

Search for modes has problems with the width of scanlines

and determining 256 or 64K colors.

Cirrus 5422:

Search for Mode has problems determing the number of bytes in a scanline.

NCR:

Search for modes has problems determining the number of bytes in a scanline

in all 256 color modes.

Oak:

The 4 color modes (55h and 57h for the 067) currently doesn't work.

Realtek:

Mode 23h (1024x768 4color) does not work.

Scroll has problems with crossing the 512K mark.

Search for Mode has problems determing the number of bytes in a scanline

VESA modes doesn't work.

S3:

The 86c924 is IDed as an 86c911A.

I hear a rumor that they are actually the same ??

Tseng 3000:

The Tseng 3000 seems to enable different amounts of memory for

different modes, thus using "larger than physical" displays

is rather problematical.

Generel Problems:

I can't tell the difference between the Sierra "Mark2" SC11482/3/4

(15 bits) and the "Mark3" SC11485/7/9 (15/16 bits) RAMDACs.

The Acumos ADAC1 seems to be misidentified as an ATT20c491/2.

With the SS24 DAC I can't tell if we are in an 256, 32768 or 16M color

mode (Search for modes).

On many cards interlaced modes are identified as app. half their

actual height by the Search for modes rutine.

Search for modes doesn't understand 2 and 4 color modes.

Some of the extended modes may only be implemented on

some otherwise similar boards, depending on make and BIOS rev.