1) int arith(int x, int y)

pushl %ebp

movl %esp,%ebp

movl 12(%ebp),%eax

movl 8(%ebp),%edx

subl %eax,%edx

imull %eax,%edx

movl %edx,%eax

movl %ebp,%esp

popl %ebp

ret

int arith(int x, int y)

{

int result = 0;

result = (x-y)*y;

return result;

}

- subl is tricky

- result is kept in %edx and not allocated on stack

- result = 0 was removed by compiler

2) int mem(int *xp, int y)

pushl %ebp

movl %esp,%ebp

movl 8(%ebp),%eax

movl 12(%ebp),%edx

movl (%eax),%ecx

movl %edx,(%eax)

movl %ecx,%eax

movl %ebp,%esp

popl %ebp

ret

int mem(int *xp, int y)

{

int result;

result = *xp;

*xp = y;

return result;

}

- result is kept in %ecx

3) int optarith(int x, int y)

pushl %ebp

movl %esp,%ebp

movl 8(%ebp),%eax

sall $4,%eax

movl %eax,%edx

subl 8(%ebp),%edx

movl 12(%ebp),%eax

testl %eax,%eax

jge .L6

addl $3,%eax

.L6:

sarl $2,%eax

addl %eax,%edx

movl %edx,%eax

movl %ebp,%esp

popl %ebp

ret

int optarith(int x, int y)

{

int result = 0;

result = x*15 + y/4;

return result;

}

- x*15 is implemented as x*16 (shift) - x

- division is implemented as right    shift, correction for negative numbers

4) int ifcode(int x, int y)

pushl %ebp

movl %esp,%ebp

movl 8(%ebp),%edx

testl %edx,%edx

jge .L8

movl $1,%eax

movl %edx,%ecx

cltd

idivl %ecx

jmp .L9

.p2align 4,,7

.L8:

movl 12(%ebp),%eax

.L9:

movl %ebp,%esp

popl %ebp

ret

int ifcode(int x, int y)

{

int result = 0;

result = x < 0 ? 1/x : y;

return result;

}

- idiv <opd>: 
%eax = (%edx:%eax)/<opd>; 
$edx = (%edx:%eax)%<opd>

- cltd: sign extends %eax to %edx

5) int whilecode(int x, int y)

pushl %ebp

movl %esp,%ebp

pushl %esi

pushl %ebx

movl 8(%ebp),%ecx

movl 12(%ebp),%esi

xorl %ebx,%ebx

testl %ecx,%ecx

je .L12

.p2align 4,,7

.L13:

movl %ecx,%edx

sarl $31,%edx

movl %esi,%eax

andl %edx,%eax

addl %eax,%ebx

addl %ecx,%ecx

jne .L13

.L12:

movl %ebx,%eax

popl %ebx

popl %esi

movl %ebp,%esp

popl %ebp

ret

int whilecode(int x, int y)

{

int result = 0;

while (x) {

int mask = (x >> 31);

result += y&mask;

x <<= 1;

}

return result;

}

- note how while loops are implemented

- loop variable is in %ecx

- result is accumulated in %ebx

- xorl %ebx, %ebx -> %ebx = 0

- left shift by one is implemented as add

6) int forcode(int x, int y)

pushl %ebp

movl %esp,%ebp

pushl %edi

pushl %esi

pushl %ebx

movl 8(%ebp),%ebx

movl 12(%ebp),%edi

xorl %edx,%edx

xorl %ecx,%ecx

cmpl %ebx,%edx

jge .L17

movl $1,%esi

.L19:

movl %esi,%eax

sall %cl,%eax

andl %edi,%eax

addl %eax,%edx

incl %ecx

cmpl %ebx,%ecx

jge .L17

cmpl $31,%ecx

jle .L19

.L17:

movl %edx,%eax

popl %ebx

popl %esi

popl %edi

movl %ebp,%esp

popl %ebp

ret

int forcode(int x, int y)

{

int result = 0;

int i;

for (i = 0; i < x && i < 32; i++)

result += y & (1 << i);

return result;

}

- loop variable is in %ecx

- result is accumulated in %edx

- %cl (lower two bytes of %ecx) is used for shifting